| Subject Code 22ES1EE101 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year I Semester Regular Examinations, January 2024

BASIC ELECTRICAL AND ELECTRONICS ENGINEERING

(Common to CSE, IT, CyS and DS)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Define KVL and KCL. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Draw the impedance triangle of series RLC Circuit. | 2M | CO-2 | BL-1 |
|  | List the speed control techniques of DC Motor. | 2M | CO-3 | BL-1 |
|  | Briefly explain the electrical characteristics of lead acid battery. | 2M | CO-4 | BL-1 |
|  | State PN junction diode and draw it’s V-I Characteristics. | 2M | CO-5 | BL-1 |

PART- B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Find the current I in the 50 Ω resistor in the circuit shown in figure. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | Find the current through the 4 Ω resistor by applying Superposition theorem. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - Derive the relation between Line Voltage and Phase Voltage in Star connected three phase system. | 5M | CO-2 | BL-2 |
|  | - Derive the relation between Line current and Phase current in Delta connected three phase system. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4. | A Voltage of 120 V at 50 Hz is applied to a resistance R in series with a capacitance C. The current drawn is 2 A and power loss in the resistance is 100 W. Calculate the (i) Resistance (ii) Capacitance and (iii) Power factor of the circuit. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | Explain the Principle of Operation and Working of single -phase transformer with neat diagrams. | 10M | CO3 | BL-2 |
| OR |  |  |  |  |
| 6. | - Derive the Expression for Generator EMF Equation. | 5M | CO3 | BL-2 |
|  | - Explain the Working of DC Motor with neat diagrams | 5M | CO3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | - Explain the principle of operation of Three phase Induction Motor. | 5M | CO4 | BL-2 |
|  | - List the differences between Squirrel cage Induction Motor and slip ring induction motor. | 5M | CO4 | BL-2 |
| OR |  |  |  |  |
| 8. | - With neat Diagrams explain the working of MCB. | 5M | CO4 | BL-2 |
|  | - Define Earthing and Explain Its Types. | 5M | CO4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | With neat Diagrams and Waveforms Explain the Operation of Full wave Rectifier. | 10M | CO5 | BL-2 |
| OR |  |  |  |  |
| 10. | Describe the working of NPN and PNP Transistors. | 10M | CO5 | BL-2 |

***