| Subject Code 22ES1EE101 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, July 2024

BASIC ELECTRICAL AND ELECTRONICS ENGINEERING

(Common to AIML, IOT and AIDS)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Find the unknown currents | 2M | CO 1 | BL 2 |
| --- | --- | --- | --- | --- |
|  | Define - Average value - Form Factor | 2M | CO 2 | BL 1 |
|  | State the working principle of transformer | 2M | CO 3 | BL 1 |
|  | Find the running speed of a 4-pole induction motor working on 50Hz supply having 3% slip | 2M | CO 4 | BL 2 |
|  | Write the applications of Diode. | 2M | CO 5 | BL 1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | a) Find the equivalent resistance between points A and B. | 5M | CO1 | BL3 |
|  | b) Find the value of ‘R’ in the circuit shown in figure below. | 5M | CO1 | BL3 |
| OR |  |  |  |  |  |
| 2. | Find current flowing through 6Ω resistor by applying superposition theorem. | 10M | CO1 | BL3 |
| UNIT-II |  |  |  |  |  |
| 3. | - Write the advantages of three phase supply. | 3M | CO2 | BL1 |
|  | - Derive the relation between line voltage and phase voltage in three phase star connected balanced supply with neat phasor diagrams. | 7M | CO2 | BL2 |
| OR |  |  |  |  |  |
| 4. | A resistance of 20Ω, inductance of 0.2 H and capacitance of 150 μF are connected in series and are fed by a 230 V, 50 Hz supply. Find Inductive reactance (XL), Capacitive reactance (XC), Impedance(Z), Power factor (p.f), active power and reactive power. | 10M | CO2 | BL3 |
| UNIT-III |  |  |  |  |  |
| 5. | - Derive the EMF equation of single-phase transformer. | 5M | CO3 | BL2 |
|  | - Write short notes on equivalent circuit of single-phase transformer. | 5M | CO3 | BL2 |
| OR |  |  |  |  |  |
| 6. | With neat diagrams explain the speed control techniques of separately excited dc motor. | 10M | CO3 | BL2 |
| UNIT-IV |  |  |  |  |  |
| 7. | - Briefly explain the speed torque slip characteristics of Induction motor. | 5M | CO4 | BL2 |
|  | - Explain the working principle of synchronous generator. | 5M | CO4 | BL2 |
| OR |  |  |  |  |  |
| 8. | - Explain the working of MCB. | 5M | CO4 | BL2 |
|  | - Give brief about types of earthing. | 5M | CO4 | BL2 |
| UNIT-V |  |  |  |  |  |
| 9. | With neat diagrams explain the V-I Characteristics of P-N junction Diode. | 10M | CO5 | BL2 |
| OR |  |  |  |  |  |
| 10. | Explain the working of transistor as an amplifier. | 10M | CO5 | BL2 |

***