| Subject Code 22ES1EE105 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, July 2024

BASIC ELECTRICAL ENGINEERING

(ECE)

Time: 3 hours                                                                                 		        Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Define transient response and steady state response. | 2M | CO1 | BL1 |
| --- | --- | --- | --- | --- |
|  | Give the expressions for symmetry and reciprocity in case of Z-parameters. | 2M | CO2 | BL1 |
|  | What is a band pass filter? | 2M | CO2 | BL1 |
|  | What are the different methods of speed control of dc shunt motor. | 2M | CO3 | BL1 |
|  | What is slip speed and synchronous speed. | 2M | CO4 | BL1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) Derive an expression for current in a series R-L circuit excited by a voltage. Assume zero initial conditions. | 5M | CO1 | BL2 |
|  | b) A series RC circuit with R=5k ohms and C = 20 micro farads has a constant-voltage source of 100V applied at t = 0; there is no initial charge on the capacitor. Obtain i, VR, Vc, for t > 0. | 5M | CO1 | BL3 |
| OR |  |  |  |  |
| 2. | Obtain the complete solution for current in a series R-L-C circuit excited by DC voltage. Assume zero initial conditions. | 10M | CO1 | BL2 |
| UNIT-II |  |  |  |  |
| 3. | a) Explain the interrelationships between Y-parameters in terms of ABCD parameters for a two-port network. | 5M | CO2 | BL2 |
|  | b) A two-port network has the following parameters: Z11=6 Ω, Z12=Z21=3 Ω and Z22=4 Ω. Calculate hybrid parameters. | 5M | CO2 | BL3 |
| OR |  |  |  |  |
| 4. | Find the Z parameters for the network shown below. | 10M | CO2 | BL3 |
| UNIT-III |  |  |  |  |
| 5. | Design an m-derived T and π network high pass filters with a cut off frequency of 10kHz, design impedance of 5ohm and m=0.4. | 10M | CO-2 | BL4 |
| OR |  |  |  |  |
| 6. | Classify Filters? Explain various types of filters with their frequency response diagram. | 10M | CO2 | BL2 |
| UNIT-IV |  |  |  |  |
| 7. | a) Derive the expression for the EMF Equation of a DC generator. | 5M | CO3 | BL3 |
|  | b) Explain the classification of dc motors with neat circuit diagrams. Also write the relationships among the currents and voltages. | 5M | CO3 | BL2 |
| OR |  |  |  |  |
| 8. | a) Explain the basic principle of operation of a dc motor. | 5M | CO3 | BL2 |
|  | b) Four pole lap connected DC generator has 600 armature conductors. The armature is rotated at 1200 rpm of the useful flux per pole is 35mWb. Calculate the amount of emf generated in the generator. | 5M | CO3 | BL3 |
| UNIT-V |  |  |  |  |
| 9. | a) A single Phase 50hz transformer has 20 primary turns and 273 secondary turns. The net cross sectional area of core is 400cm2. If the primary winding is connected to 230V supply, find (i) peak value of flux density in the core (ii) Voltage induced in the secondary winding. | 5M | CO4 | BL3 |
|  | b) Explain the constructional differences between squirrel cage and slip ring induction motor. | 5M | CO4 | BL2 |
| OR |  |  |  |  |
| 10. | a) Derive an EMF Equation of a Single-Phase Transformer. | 5M | CO4 | BL3 |
|  | b) Explain the Principle of operation of Alternators. | 5M | CO4 | BL2 |

***