| Subject Code 22ES1EE106 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, July 2024

CIRCUIT THEORY

(EIE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define Kirchhoff’s Voltage Law with example. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Differentiate Mesh and Loop. | 2M | CO-1 | BL-2 |
|  | What is meant by real and reactive powers. | 2M | CO-2 | BL-2 |
|  | Differentiate tie set and cut set of a tree. | 2M | CO-4 | BL-2 |
|  | When two coils having self-inductances of L1 and L2 are coupled through a mutual inductance M, what is value of the coefficient of coupling k . | 2M | CO-3 | BL-3 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Find the voltage across the 4 ohms resistor shown in network. | 5M | CO-1 | BL-3 |
|  | - Classify the circuit elements in detail and explain | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Calculate the equivalent Rab for the circuit shown in figure. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - Calculate the current i0 using nodal analysis | 5M | CO-1 | BL-3 |
|  | - Find the branch currents i1, i2 and i3 using mesh analysis | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 4. | Find the current i shown in figure. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - Define and obtain the relation between active, reactive and complex powers in a circuit in frequency domain. | 5M | CO-2 | BL-2 |
|  | - Derive the equation for bandwidth of series resonance circuit. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6. | The waveform shown in Figure .find the peak factor, form factor and the amount of average power absorbed by 9Ω resistor. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | Use the superposition theorem to find v0 | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8. | State and explain Thevenin’s theorem. Enumerate the relation between Thevenin’s and Norton’s equivalent circuits. | 10M | CO-2 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | - Find the equivalent inductance of the network. | 5M | CO-3 | BL-2 |
|  | - State and explain Faraday’s laws of electromagnetic induction. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 10. | Two coils having 3000 and 2000 turns are wound on a magnetic ring 60% of the flux produced in the first coil links with the second coil. A current of 3A produce a flux of 0.5 mwb in the first coil and 0.3 mwb in the second coil. Determine the mutual inductance and coefficient of coupling. | 10M | CO-3 | BL-3 |

***