| Subject Code 22ES1ME101 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, July 2024

ENGINEERING MECHANICS

(ME)

Time: 3 hours                                                                                 		        Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Define equilibrium of motion. Write the equations for equilibrium. | 2M | CO1 | BL1 |
| --- | --- | --- | --- | --- |
|  | Define centroid and centre of gravity. | 2M | CO2 | BL1 |
|  | What is polar moment of inertia? | 2M | CO3 | BL1 |
|  | State the D`Alembert`s principle | 2M | CO4 | BL1 |
|  | State the principle of impulse-momentum. | 2M | CO5 | BL1 |

PART-B

5X8=40M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - What do you understand by the term ‘Couple’? Discuss the characteristics of a couple. | 5M | CO1 | BL1 |
|  | - Figure shows two vertical forces and a couple of moment 2000 N-m acting on a horizontal rod which is fixed at end A. Determine the resultant of the system. | 5M | CO1 | BL3 |
| OR |  |  |  |  |  |
| 2. | - State and Prove Lami’s theorem. | 5M | CO1 | BL1 |
|  | b) Three cylinders weighting 100 N each and of 80 mm diameter are placed in a channel of 180 mm width as shown in figure below. Find the support reactions. | 5M | CO1 | BL3 |
| UNIT-II |  |  |  |  |  |
| 3. | Determine the magnitude and direction of the friction force acting on the 100-kg block shown in the figure if, first, P = 500 N and, second, P = 100 N are applied. The coefficient of static friction is 0.20, and the coefficient of kinetic friction is 0.17. The forces are applied with the block initially at rest. | 10M | CO2 | BL4 |
| OR |  |  |  |  |  |
| 4. | Determine the centroid of the area bounded by the x-axis, the line x=a and the parabola y2=kx as shown in the figure. | 10M | CO2 | BL4 |
| UNIT-III |  |  |  |  |  |
| 5. | - State and prove perpendicular axis theorem for area moment of inertia. | 5M | CO3 | BL2 |
|  | b) Figure. shows the cross-section of a cast iron beam. Determine the area moments of inertia of the section about horizontal and vertical axes passing through the centroid of the section. | 5M | CO3 | BL4 |
| OR |  |  |  |  |  |
| 6. | Determine the Mass moment of inertia of a solid cone whose height is H and base radius is R. | 10M | CO3 | BL4 |
| UNIT-IV |  |  |  |  |  |
| 7. | A ball is thrown so that it just clears a 8.5 m wall 32m away. If it left the 1.5m above the ground and at an angle of 400 to the horizontal. What is the initial velocity of the ball. | 10M | CO4 | BL3 |
| OR |  |  |  |  |  |
| 8. | A light rope passing round a pulley weightless and frictionless has two masses 8 kg and 6 kg attached to its ends. If the rope does not slip as the pulley rotates, determine the acceleration of the two masses and tension in the rope. | 10M | CO4 | BL4 |
| UNIT-V |  |  |  |  |  |
| 9. | A body weighing 300N pushed up a 30º plane by a 400N force acting parallel to the plane. If the initial velocity is 1.5 m/sec and μ = 0.2, what velocity will the body have after moving 6m? | 10M | CO4 | BL4 |
| OR |  |  |  |  |  |
| 10. | Determine the velocity of body A and body B after 5 sec if the system is released from rest. Neglect friction and the inertia of the pulleys. | 10M | CO4 | BL3 |

***