| Subject Code 22ES1EE103 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, July 2024

FUNDAMENTALS OF ELECTRICAL AND ELECTRONICS ENGINEERING

(AE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Define power and power factor of a given electrical circuit. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What is the use of yoke in a DC machine? | 2M | CO-3 | BL-2 |
|  | Define the term turns ratio of a transformer. | 2M | CO-2 | BL-1 |
|  | Describe slip of three phase induction motor. | 2M | CO-3 | BL-1 |
|  | Sketch the symbol of NOR gate with the truth table. | 2M | CO-4 | BL-3 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | a) State and illustrate Kirchhoff’s laws using an examples. | 4M | CO-1 | BL-2 |
|  | b) Find the average and effective values of the sine wave form shown in figure below. | 6M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
| 2. | a) Write short notes on RMS Value and form factor of AC waveforms. | 4M | CO-1 | BL-3 |
|  | b) A pure resistance of 50 ohms is in series with a pure capacitance of 100 microfarads. This series combination is connected across 100V 50Hz supply. Find: i. The impedance ii. Current iii. Power factor iv. Phase angle | 6M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3. | a) Derive the EMF equation of DC generator. | 6M | CO-3 | BL-3 |
|  | b) Classify DC Motors in detail. | 4M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 4. | a) Illustrate the principle of operation DC generator with neat sketch. | 4M | CO-3 | BL-3 |
|  | b) A 6-pole generator has a lap-wound armature with 40 slots with 20 conductors per slot. The flux per pole is 25mWb. Calculate the speed at which the machine must be driven to generate an e.m.f. of 300 V. | 6M | CO-3 | BL-4 |
| UNIT-III |  |  |  |  |  |
| 5. | a) Derive the EMF equation of a single phase transformer? | 5M | CO-2 | BL-2 |
|  | b) Illustrate the procedure of determining constant losses on a 1-φ transformer with neat sketch. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 6. | a) Explain the principle of operation of 1-φ transformer. | 4M | CO-2 | BL-2 |
|  | b) In a 50 KVA, 2000/ 200V, single-phase transformer, the iron and full load copper losses are 350 W and 400 W respectively. Calculate the efficiency at unity power factor on (i) full-load (ii) half-full load. | 6M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7. | a) Draw and explain the torque-slip characteristics of Induction motor. | 4M | CO-3 | BL-3 |
|  | b) Derive the expression for torque of an induction motor and obtain the conditions for maximum torque. | 6M | CO-3 | BL-4 |
| OR |  |  |  |  |  |
| 8. | Discuss the process of finding the regulation of an alternator by synchronous impedance method with neat sketch. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9. | a) Illustrate the operation of PN junction diode with its I-V characteristics. | 6M | CO-4 | BL-2 |
|  | b) Describe the AND, OR, NAND logic gates with truth tables. | 4M | CO-4 | BL-3 |
| OR |  |  |  |  |  |
| 10. | With necessary diagrams, Describe BJT operation as a linear device and BJT operation as a switch. | 10M | CO-4 | BL-3 |

***