| Subject Code 22ES1EE07 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, July 2024

NETWORK ANALYSIS

(EEE)

Time: 3 hours                                                                                 		      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | What is the significance of phase sequence? | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Formulate Laplace Transform of step and ramp signals. | 2M | CO-2 | BL-2 |
|  | Define poles and Zeros. | 2M | CO-3 | BL-1 |
|  | Write the Conditions for Reciprocity and Symmetry. | 2M | CO-3 | BL-2 |
|  | Write the Fourier trigonometric expression of periodic function f(t). | 2M | CO-5 | BL-2 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) Develop the relations between phase quantities and line quantities in a 3-phase balanced star connected system. | 5M | CO-1 | BL-2 |
|  | b) Three impedances each of (3–j4) Ω are connected in delta connection across a 3-phase, 230V balanced supply. Evaluate line and phase currents in the delta connected load. Formulate the value of power delivered to the load. Consider RYB phase sequence. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | a) The impedances ZR= j30 Ω, ZY= j3 Ω, ZB = –j3 Ω are connected in the form of a star connected 4 wire unbalanced system and it is connected to a 400 V, 3-phase supply. Find the line and phase cuurents. | 3M | CO-1 | BL-3 |
|  | b) Two-wattmeter method is used to measure power in a 3-phase load. The wattmeter readings are 400W and –35W. Calculate (i) Total active power (ii) Power factor and (iii) Reactive power of three phase load. | 7M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | a) Derive the transient response of a series RL circuit with dc input. Sketch the variation of current through the inductor. | 7M | CO-2 | BL-2 |
|  | b) Circuit as shown for t>0. At t=0- the network was unenergized. Write the 2nd order differential equation. Find the roots of the equation. | 3M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | The switch is in position 1 for long time and moved to position 2 at t= 0, find current i(t) for t>0. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | a) List the properties of Driving point function. | 5M | CO-3 | BL-2 |
|  | b) Evaluate the driving point impedance and the pole-zero plot of the given network. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | A transform voltage is given by VS=2S(S+1)(S+4) . - Find pole- zeros of given V(s). - Plot pole -zero in S-plane. - Obtain the Laplace inverse of V(s). | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | a) Describe the interconnection of two two port networks in cascade. | 5M | CO-3 | BL-2 |
|  | b) For the two port network shown in figure compute h-parameters. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8. | - Evaluate Z-parameters of the network shown below. | 7M | CO-3 | BL-3 |
|  | b) Express Y parameters in terms of Z parameters. | 3M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | - Design the L and C of symmetrical T network of constant ‘k’ low pass filter. | 5M | CO-4 | BL-4 |
|  | - Write a short note on types of filters. | 5M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 10. | a) Determine the Fourier coefficients a0 , an , bn of given wave form. One cycle of period T=2π. b) Determine the average power observed by the circuit due to periodic function Vt=vdc+n=1∞vncos(nωt-ϴn) it=idc+n=1∞incos(nωt-Фn) | 10M | CO-5 | BL-3 |

***