| Subject Code 22PC1EI101 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, July 2024

SENSORS AND TRANSDUCERS

(EIE)

Time: 3 hours                                                                                 		       Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Differentiate a sensor and a Transducer. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Explain the working principle of Light-Dependent Resistors. | 2M | CO-1 | BL-1 |
|  | What are Eddy current sensors. | 2M | CO-1 | BL-1 |
|  | Define Hall Effect. | 2M | CO-1 | BL-1 |
|  | Explain the working principle of Quartz digital thermometers. | 2M | CO-1 | BL-1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | Explain and Classify the Transducers in detail? Write examples of Each. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2. | With a neat sketch briefly explain input and output configuration of Measurement System. Explain Methods of Correction of above system. | 10M | CO-1 | BL-1 |
| UNIT-II |  |  |  |  |  |
| 3. | - Define RTD. With a neat sketch explain the working principle of RTD. | 5M | CO-2 | BL-1 |
|  | - Explain about Resistive Hygrometers with a neat sketch. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 4. | ### Define Gauge factor. Derive expression for it. Calculate the gauge factor of strain gauge, if the value of resistance is 152 Ohms. Which changes by 5 Ohms for 5000 micro strain. | 10M | CO-4 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5. | a) What is LVDT. Explain its characteristics. | 5M | CO-2 | BL-1 |
|  | - Explain about capacitive Touch sensor with a neat sketch. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 6. | - With a neat sketch explain the Reluctance variation sensor? | 5M | CO-2 | BL-1 |
|  | - Explain about Differential capacitor sensor with a neat sketch. | 5M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |  |
| 7. | - What is Piezoelectric effect. What are the materials used for Piezoelectric Sensors. | 5M | CO-3 | BL-2 |
|  | - Differentiate Photovoltaic Sensors and Pyroelectric Sensors. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 8. | Define Seeback effect. What are the materials used for thermocouple. Briefly explain the types of Thermocouples. | 10M | CO-3 | BL-1 |
| UNIT-V |  |  |  |  |  |
| 9. | - Write a short note on SAW sensors. | 5M | CO-1 | BL-1 |
|  | - What is the principle of Encoder. Write a brief note on absolute position encoders. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 10. | - With a neat sketch explain about vibrating cylinder Sensors. | 5M | CO-2 | BL-1 |
|  | - Write a short note on MEMS sensors. | 5M | CO-1 | BL-2 |

***