| Subject Code 22PC1CY201 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January / February 2024

COMPUTER NETWORKS AND ETHICAL HACKING

(CSE-CyS)

Time: 3Hrs											      Max.Marks:60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

|  | List out the five components of a data communication system. | 2M | CO1 | BL2 |
| --- | --- | --- | --- | --- |
|  | Differentiate between interior and exterior routing protocols. | 2M | CO2 | BL4 |
|  | What is the purpose of the DNS and DDNS protocols. | 2M | CO3 | BL2 |
|  | Write the various Foot printing countermeasures. | 2M | CO4 | BL2 |
|  | Draw the vulnerability management life cycle. | 2M | CO5 | BL2 |

PART- B

5X10=50M

| UNIT-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | a) The OSI reference model defines seven protocol layers, each of which is responsible for specific range of functions. By considering this model, explain the main functions performed by a protocol operating at: (i) Datalink Layer (ii) Transport Layer (iii) Presentation Layer | 5M | CO-1 | BL-2 |
|  | b) A bit stream 10011101 is transmitted using the standard CRC method described in the text. The generator polynomial is x3 + 1. Show the actual bit string transmitted. Suppose the third bit from the left is inverted during transmission. Show that this error is detected at the receiver's end. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
| 2. | What is meant by topology? Explain the different types of topologies and compare each topology with the other with merits and De merits | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |  |
| 3. | a) With the help suitable example explain the Open Shortest Path First (OSPF) routing protocol. | 5M | CO-2 | BL-2 |
|  | b) Describe the DHCP protocol delivery mechanism. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 4. | Draw and explain the IPv4 header format and compare that how IPv6 is different from IPv4. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |  |
| 5. | a) With the help of neat sketch explain the Architecture of E-Mail. | 5M | CO-3 | BL-2 |
|  | b) What are the different request types available in HTTP? Explain. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 6 | Write the structure of TCP pseudo header and explain how it is used in checksum calculation. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7 | a) Enumerate different phases of Ethical hacking? Explain each in detail. | 5M | CO-4 | BL-4 |
|  | b) What is meant by footprinting? What is the purpose of the foot printing and what information to be gathered about target system during foot printing? | 5M | CO-4 | BL-4 |
| OR |  |  |  |  |  |
| 8 | What is meant by scanning? How host discovery and port scanning can be done using NMAP protocol. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9 | a) What is enumeration? What information can be enumerated by intruders? Explain the DNS enumeration technique. | 5M | CO-5 | BL-2 |
|  | b) How vulnerability scanning can be done using NMAP protocol? | 5M | CO-5 | BL-3 |
| OR |  |  |  |  |  |
| 10 | Write a brief note on the following databases. - CVSS - CWE | 10M | CO-5 | BL-2 |

***