| Subject Code 22PC1EC211 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January /February 2024

DIGITAL LOGIC DESIGN

(Common to CSE, IT, CyS, and IoT)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2 =10M

|  | Convert (10110)2 to Gray code and (110101)G to binary number. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Write the truth table of Ex-OR Gate. | 2M | CO-2 | BL-1 |
|  | Differentiate multiplexer and de-multiplexer. | 2M | CO-3 | BL-2 |
|  | What are PLAs and PALs? | 2M | CO-4 | BL-2 |
|  | What is the difference between synchronous and asynchronous counter. | 2M | CO-5 | BL-2 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Convert the following. i) (53.625)10 to ( ? )2 ii) (3FD)16 to ( ? )2 iii) (A69.8)16 to ( ? )10 | 5M | CO-1 | BL-2 |
|  | - Perform the decimal subtraction in 8-4-2-1 BCD using 9's complement i) Subtract 79 from 26 ii) Subtract 748 from 983 | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | a) Given the 8 bit data word 11011011, generate the 12 bit composite word for the Hamming code that corrects and detects single errors | 5M | CO-1 | BL-2 |
|  | b) Explain about even and odd parity check with an example, what is the drawback | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | a) Obtain the simplified expression in POS (product of sums) of F(w,x,y,z)=π(1,2,4,7,12,14,15) using K-maps | 5M | CO-2 | BL-3 |
|  | b) Derive the sum of minterms for f(a,b,c,d)=a′b+ab′d+c′d. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | Obtain the simplified expression in sum of products for the following Boolean function. a) F(A,B,C,D) = ∑(2,3,12,13,14,15). b) BDE+B′C′D+CDE+A′B′CE+A′B′C+B′C′D′E′ | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | Draw and explain the operation of 3 to 8 decoder | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | a) Define a multiplexer? Draw a 4:1 multiplexer for the function. f(a,b,c,d)=∑(0, 4,5,10,11,12,15) b) Design a full binary adder with two half adders and an OR gate. | 10M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | Realize D-latch using S-R latch. How is it different from D-flip flop? Draw the circuit using NAND gates and explain. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8. | Explain the working of a master-slave JK flip flop. State its advantages. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | - Discuss the key steps involved in designing a finite state machine. Illustrate the design process with an example, including the specification, state diagram, state table, and state assignment. | 5M | CO-5 | BL-2 |
|  | - Design a 3-bit counter using T flip flops. | 5M | CO-5 | BL-3 |
| OR |  |  |  |  |
| 10. | What is the difference between serial and parallel transfer? Explain how to convert serial data to parallel and parallel data to serial. What type of register is needed? | 10M | CO-5 | BL-2 |

***