| Subject Code 22PC1EE201 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January/February 2024

ELECTRICAL MACHINES - I

(EEE)

Time: 3 hours                                                                                 		              Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART – A

5 X2=10M

|  | State Faraday’s Laws of Electromagnetic Induction | 2M | CO1 | L2 |
| --- | --- | --- | --- | --- |
|  | Write the differences between Magnetic & Electric Circuit | 2M | CO1 | L1 |
|  | Write the applications of Lap and wave winding in DC-Generator? | 2M | CO2 | L1 |
|  | Explain why the rating of transformer is expressed in KVA? | 2M | CO1 | L1 |
|  | What is the use of On-Load tap changing transformer? | 2M | CO3 | L1 |

PART- B

5X10=50M

| UNIT-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | a) Prove that the energy and Co-energy in a linear magnetic system are given by identical expressions. | 5M | CO-1 | L3 |
|  | b) An 8-pole generator has an output of 200Amps at 500V, the lap connected armature has 1280 conductors and 160 commutator segments, if the brushes are advanced 4-segments from the no-load neutral axis, estimate the armature demagnetizing and cross magnetizing ampere turn per pole? | 5M | CO-1 | L4 |
| OR |  |  |  |  |  |
| 2. | With neat sketch explain the construction and working principle of DC Machine. | 10M | CO-1 | L2 |
| UNIT-II |  |  |  |  |  |
| 3. | a) The magnetization characteristics for a 4 pole, 110V, 1000rpm shunt generator is as follows Field current: 0 0.5 1 1.5 2 2.5 3 O.C Voltage: 5 50 85 102 112 116 120V Determine the critical resistance and critical speed. | 5M | CO-2 | L4 |
|  | b) Explain about commutation and any one method to improve commutation? | 5M | CO-2 | L2 |
| OR |  |  |  |  |  |
| 4. | With necessary equations draw the internal, external characteristics of DC shunt, series, long shunt and short shut compound generator. | 10M | CO-3 | L2 |
| UNIT-III |  |  |  |  |  |
| 5. | a) Derive the torque equation of a DC Motor. | 5M | CO-1 | L3 |
|  | b) With necessary equations explain how above and below rated speed of DC Shunt motor is controlled. | 5M | CO-2 | L3 |
| OR |  |  |  |  |  |
| 6. | Describe the Field’s Test on DC motor to determine the performance characteristics of DC motor | 10M | CO-4 | L2 |
| UNIT-IV |  |  |  |  |  |
| 7. | a) Explain the working principle of transformer and draw the phasor diagram for RL load. | 5M | CO-1 | L2 |
|  | b) A 5KVA 200/1000V, 50Hz, 1-Ø transformer gave the following test results: O.C Test (LV Side): 200V, 1.2A, 90W S.C Test (H.V Side): 50V, 5A, 110W. Calculate the parameters of the equivalent circuit referred to the L.V Side. | 5M | CO-4 | L4 |
| OR |  |  |  |  |  |
| 8. | Find the all-day efficiency of 500KVA distribution transformer whose copper loss and iron loss at full load are 4.5kW and 3.5kW. during the day of 24hours it is loaded as under No of Hours Loading in kW Power factor 6 400 0.8 10 300 0.75 4 100 0.8 4 0 --- | 10M | CO-4 | L4 |
| UNIT-V |  |  |  |  |  |
| 9. | a) Prove that saving of copper in autotransformer is more compared to 2 winding transformer. | 5M | CO-1 | L3 |
|  | b) Explain the importance and conditions for parallel connection of 1-Ø transformers? | 5M | CO-2 | L2 |
| OR |  |  |  |  |  |
| 10. | With necessary equations prove that 3- Ø supply is converted into 2-Ø supply using Scott connection of transformers. | 10M | CO-2 | L2 |

***