| Subject Code 22PC1EE202 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January /February 2024

ELECTROMAGNETIC FIELDS

(EEE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Explain the Gauss law. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What is the difference between conduction and convection current density? | 2M | CO-1 | BL-2 |
|  | Write alternate forms of Biot savart’s law. | 2M | CO-2 | BL-1 |
|  | Write the expression for force on a differential current element in magnetic fields. | 2M | CO-3 | BL-1 |
|  | Differentiate between the statically and dynamically induced emfs. | 2M | CO-4 | BL-2 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - A point charge of 10 C is located at (1,1,2) in free space, while a charge of 1 C is at (4,1,3). Find the vector force between the charges. | 5M | CO-1 | BL-3 |
|  | - Obtain an expression for electric field intensity due to an infinite sheet of charge having a uniform surface charge density of ρs. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | - State and explain Coulomb's law for the vector force between the two point-charges in free space. | 5M | CO-1 | BL-2 |
|  | - Point charges of 1 μC and -1 μC are located at (0,0,1) m and (0,0,-1) m respectively in free space. Find the potential at (0,3,4) m | 5M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - Obtain the expression for electric potential due to a dipole. | 5M | CO-1 | BL-2 |
|  | - A potential distribution is given by V = 5y2 + 10x. Determine the electric field intensity E at points (0, 0, 0) and (10, 2, 0). | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 4. | - Derive the expression for equation of continuity for steady currents. | 5M | CO-1 | BL-3 |
|  | - Derive the boundary conditions between two perfect dielectric medium. | 5M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - State and explain Biot-Savart’s Law. | 6M | CO-2 | BL-2 |
|  | - Explain the significance of Maxwell’s second equation. | 4M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6. | State and explain Ampere’s circuital law and derive ∇ X H= J. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | - Derive the expression for torque on a current loop placed in a magnetic field. | 5M | CO-3 | BL-3 |
|  | - A point charge of Q = -1.2 C has velocity V=5ax+2ay -3az m/s. Find the magnitude of the force exerted on the charge if both E=-18ax+5ay -10az V/m and B=-4ax+4ay+3az T are present simultaneously. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8. | Two infinitely long parallel conductors are separated by a distance ‘d’. Find the force per unit length exerted by one of the conductors on the other if the currents in the two conductors are I1 and I2. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | - Derive the expression for Displacement current density. | 5M | CO-4 | BL-3 |
|  | - In a given lossy dielectric medium, conduction current density Jc=0.02 sin109 t A/m2. Find the displacement current density, if σ =103 S /m and εr = 6.5. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10. | State Maxwell’s equation for static fields. Explain how they are modified for time varying electric and magnetic fields. | 10M | CO-4 | BL-2 |

***