| Subject Code 22PC1EC201 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January /February 2024

ELECTRONIC CIRCUIT ANALYSIS

(ECE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Justify why gain of an amplifier reduces at lower frequencies. | 2M | CO1 | BL3 |
| --- | --- | --- | --- | --- |
|  | Draw the circuit diagram of Darlington amplifier. | 2M | CO2 | BL1 |
|  | What are the advantages of negative feedback? | 2M | CO3 | BL2 |
|  | List the basic conditions for oscillation. | 2M | CO4 | BL1 |
|  | Draw the circuit diagram of Source follower. | 2M | CO5 | BL2 |

PART- B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Tabulate typical values of h-parameters of CE, CB and CC configurations. | 5M | CO-1 | BL-1 |
|  | - Draw the circuit diagram of CB amplifier and explain its operation. Also draw its equivalent hybrid model. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Draw the equivalent circuit of CE amplifier with Emitter resistance. Also perform analysis of this circuit. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3. | - Explain different classification of amplifiers. | 5M | CO-2 | BL-2 |
|  | - With suitable diagrams, explain briefly about different coupling schemes used in amplifiers. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4. | Draw and explain the operation of cascode amplifier. Also derive the expression for its input resistance and voltage gain. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5. | - Derive the expression for closed loop gain of a general amplifier structure with negative feedback. | 5M | CO-3 | BL-4 |
|  | - A current-series feedback amplifier employs a basic amplifier with input and output resistances of 10kΩ each and gain A=2000. If feedback factor β=0.01, find the gain Af, Rif, Rof of closed loop amplifier. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | Draw the block diagram of Voltage series feedback amplifier. Also derive the expression for its input and output resistances. | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | - Draw the circuit diagram of RC phase shift oscillator and explain its operation. | 5M | CO-4 | BL-2 |
|  | - Draw and explain the equivalent circuit of crystal oscillator. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | Draw and explain the operation of Hartley oscillator. Also derive the expression for its frequency of oscillation. | 10M | CO-4 | BL-4 |
| UNIT-V |  |  |  |  |
| 9. | - Compare and contrast CS, CD, and CG JFET amplifiers. | 5M | CO-5 | BL-4 |
|  | - Draw and explain the small signal model of MOSFET. | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10. | Draw the circuit diagram of Common source amplifier with source resistance and explain its operation. Also draw its small signal equivalent model. | 10M | CO-5 | BL-3 |

***