| Subject Code 22PC1EI202 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January / February 2024

ELECTRONIC MEASUREMENTS

(EIE)

Time: 3 hours                                                                                 		                Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Define Sensitivity and Resolution. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Identify the factors that may lead to inaccuracies in measurement by AC bridges. | 2M | CO-2 | BL-2 |
|  | How an ammeter can be changed into a voltmeter. | 2M | CO-3 | BL-3 |
|  | Recognize the advantages of wave analyzers. | 2M | CO-4 | BL-2 |
|  | Differentiate dual trace and dual beam CRO. | 2M | CO-5 | BL-4 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | - The following 10 observations were recorded when measuring a voltage: S.No; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10. Volts; 41.7; 42; 41.8; 42; 42.1; 41.9; 42.5; 42; 41.9; 41.8. Find (i)mean (ii) standard deviation (iii) probable error of one reading. - Describe primary and secondary standards | 5M 5M | CO-1 CO-1 | BL-4 BL-2 |
| OR |  |  |  |  |
|  | Explain static and dynamic characteristics of measurement systems. | 10M | CO-1 | BL-2 |
| UNIT-2 |  |  |  |  |
|  | a) Describe the working principle of Schering bridge and derive an expression for measurement of unknown capacitor. b) Justify the need of Wagner’s ground connection in Bridges. | 6M 4M | CO-2 CO-2 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | Explain with the help of circuit diagram, how kelvin bridge is used to measure low resistance. Derive the necessary relationship used to determine unknown resistance in terms of known resistance of the bridge circuit | 10M | CO-2 | BL-2 |
| UNIT-3 |  |  |  |  |
|  | - Explain the operation of Ramp type DVM. - Discuss the working of Watt-hour meter. | 5M 5M | CO-3 CO-3 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | - Describe the basic principle and operation of AC voltmeter using rectifier. - Explain about calibration of voltmeter and ammeter. | 5M 5M | CO-3 CO-3 | BL-2 BL-2 |
| UNIT-4 |  |  |  |  |
|  | - Explain basic principle and operation of digital frequency meter. - Describe the working of spectrum analyzer. | 5M 5M | CO-4 CO-5 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | ## Illustrate different Modes of Operation of Electronic Counter. | 10M | CO-5 | BL-3 |
| UNIT-5 |  |  |  |  |
|  | - Describe measurement of frequency and phase using CRO. - Discuss about 14 segment and Dot Matrix display devices. | 5M 5M | CO-5 CO-5 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | Sketch the block diagram of basic CRO and explain each block. | 10M | CO-5 | BL-3 |

***