| Subject Code 22PC1CE203 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January/February 2024

FLUID MECHANICS

(CE)

Time: 3Hrs											      Max.Marks:60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Define fluid Pressure. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Differentiate Steady and Unsteady flow. | 2M | CO-1 | BL-2 |
|  | Classify manometers. | 2M | CO-1 | BL-2 |
|  | Name the non-dimensional number that is used to differentiate between laminar flow and turbulent flow and write its equations. | 2M | CO-2 | BL-1 |
|  | Define Magnus effect in boundary layer theory. | 2M | CO-1 | BL-1 |

PART- B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Explain the terms. - The concept of Continuum. - Density - Specific volume. | 5M | CO-1 | BL-2 |
|  | b) The right limb of simple U-tube manometer is open to atmosphere. The left limb is connected to a pipe in which fluid of specific gravity 0.9 is flowing. The manometer contains mercury. The center of the pipe is 12 cm below the level of mercury in the right limb and 5cm above the level of mercury in left limb. Find the pressure in pipe. Draw neat diagram. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 2. | a) Differentiate absolute pressure, gauge pressure and Vacuum pressure. b) Derive the expression for total pressure for a vertical plane surface submerged in liquid. | 6M 4M | CO-1 CO-2 | BL-2 BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - Explain the terms. - Uniform and Non uniform flows - Rotational and irrotational flows. - 1-D, 2-D and 3-D flow, | 5M | CO-1 | BL-2 |
|  | b) Differentiate velocity potential function and stream function. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 4. | a) The velocity vector in a fluid flow is given by4x3i-10x2yj+2tk. Find the velocity and acceleration of a fluid particle at (2,1,3)at time t=1. b) Explain Bernoulli’s equation and its applications. | 5M 5M | CO-3 CO-1 | BL-3 BL-2 |
| UNIT-III |  |  |  |  |
| 5. | a) Derive the expression for a discharge through a rectangular notch. | 4M | CO-2 | BL-3 |
|  | b) Determine the height of a rectangular weir of length 6m to be built across a rectangular channel. The maximum depth of water on the upstream side of the weir is 1.8 m and the discharge is 2000 lits/s. Take cd=0.6 and neglect end contractions. | 6M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | Derive the expressions for Hydraulic coefficients of orifice. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | a) Differentiate pipes in series and pipes in parallel. | 3M | CO-1 | BL-2 |
|  | b) An oil of viscosity 0.1 N.S/m2 and relative density 0.9 is flowing through a circular pipe of diameter 50 mm and length 300 m. The discharge through the pipe is 3.5 lit/s. Find the pressure drop in a length of 300 m and the shear stress at the pipe wall. | 7M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8. | Derive the expression for velocity distribution and shear stress for laminar flow through circular pipe. | 10M | CO-2 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | - Explain the terms Drag and Lift with equations. | 4M | CO-1 | BL-2 |
|  | - What is boundary layer separation and under what conditions it takes place? How to control its formation. | 6M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 10. | Find the displacement thickness, momentum thickness and energy thickness for velocity distribution in boundary layer is given by uU=yδ, where ‘u’ is velocity at distance at y from the plate and u=U at y = δ where δ is boundary layer thickness. Also calculate ratio of displacement thickness to momentum thickness. | 10M | CO-3 | BL-3 |

***