| Subject Code 22ES1EE201 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January /February 2024

FUNDAMENTALS OF ELECTRICAL ENGINEERING

(EIE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | What is the condition for maximum efficiency in DC machines? | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Define slip speed. | 2M | CO-2 | BL-1 |
|  | What is the objective of transient analysis? | 2M | CO-3 | BL-2 |
|  | Write the condition for symmetry and reciprocity for ABCD parameters. | 2M | CO-4 | BL-2 |
|  | Define characteristic Impedance. | 2M | CO-4 | BL-1 |

PART- B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | # a) Enumerate all parts of dc machine and state the function of each part. | 5M | CO-1 | BL-2 |
|  | b) An 8-pole lap wound DC generator armature has 960 conductors, a flux of 40 mWb and a speed of 400 rpm calculate the emf generated on open circuit. If the same armature is wave wound, at what speed must it be driven to generate 400 V? | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | a) Describe Swinburne’s test to find out the efficiency of a DC machine with the help of a neat circuit diagram. | 5M | CO-1 | BL-2 |
|  | b) A 200V dc shunt motor running at 1000rpmtakes an armature current of 17.5A. It is required to reduce the speed to 600rpm.What must be the value of resistance to be inserted in the armature circuit if the original armature resistance is 0.4ohm? Take armature current to be constant during this process. | 5M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | a) A single-phase transformer has 180 and 90 turns respectively in its secondary and primary windings. The respective resistances are 0.233Ω and 0.067Ω. Calculate the equivalent resistance of (i) primary referred to secondary (ii) secondary referred to primary (iii) total resistance referred to primary. | 5M | CO-2 | BL-3 |
|  | b) Explain the working principle of a 3-phase Induction Motor and draw the torque-slip characteristics. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4 | a) Discuss the principle of operation of an alternator. | 5M | CO-2 | BL-2 |
|  | b) Explain the operation of practical transformer and draw the phasor diagrams by considering the winding resistance and leakage reactance for inductive load. | 5M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5 | a) A series RLC circuit comprising R=10Ω, L=0.5H and C=1.4µF is excited by a constant voltage source of 100V. Obtain the expression for the current. Assume that the circuit is relaxed initially. | 5M | CO-3 | BL-3 |
|  | b) A series RL circuit with R = 30Ω and L = 15 H has a constant voltage V = 60V applied at t = 0 as shown in fig. Determine the current I, the voltage across resistor and the voltage across the inductor | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6 | In the Figure, determine complete solution for current, when switch ‘K’ is closed at t = 0 for applied voltage V(𝑡) = 200V. Derive the expression for the current. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | Find ABCD parameters of a given network and mention the conditions for symmetry. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8 | - The Z – parameters of a two-port network are Z11 = 20Ω, Z12= Z21= 10Ω, Z22 = 30Ω. Determine Y-parameters. | 5M | CO-4 | BL-3 |
|  | - Find the Y – parameters for the following circuit. | 5M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | Design a constant-K low pass T and π section filter having cut-off frequency of 4kHz and nominal impedance of 500ohm. | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10 | - Explain about m derived high pass filter. | 5M | CO-4 | BL-2 |
|  | - Derive the expression for characteristic impedance of a T- network. | 5M | CO-4 | BL-2 |

***