| Subject Code 22PC1AE205 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January/February 2024

FUNDAMENTALS OF THERMODYNAMICS

(AE)

Time: 3 hours                                                                                 		              Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Define state, property, process, and cycle. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What do you mean by “Perpetual motion machine of first kind-PMM 1” ? | 2M | CO-1 | BL-2 |
|  | State kelvin-Planck statement. | 2M | CO-2 | BL-2 |
|  | Draw P-V diagram for water and a pure substance other than water. | 2M | CO-2 | BL-2 |
|  | Name the process in Diesel cycle and draw the p-v diagram. | 2M | CO-3 | BL-2 |

PART- B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - What is a quasi-static process? Explain it with a neat sketch | 5M | CO-1 | BL-2 |
|  | - Define a thermodynamic system. Differentiate between open system, closed system and an isolated system. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | In a closed system 150 kJ of work is supplied. If the initial volume is 0.6 m3 and pressure of the system changes as p = 8 – 4V, where p is in bar and V is in m3, determine the final volume and pressure of the system. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - a) A closed system of constant volume experiences a temperature rise of 25°C when a certain process occurs. The heat transferred in the process is 30 kJ. The specific heat at constant volume for the pure substance comprising the system is 1.2 kJ/kg°C, and the system contains 2.5 kg of this substance. Determine: (i) The change in internal energy; (ii) The work done. | 5M | CO-1 | BL-2 |
|  | - State the First Law of Thermodynamics and prove that for a non-flow process, it leads to the energy equation Q = ΔU + W. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 4. | Write down the general energy equation for steady flow system and simplify when applied for the following systems: (i) Centrifugal water pump (ii) Reciprocating air compressor (iii) Steam nozzle (iv) Steam turbine (v) Gas turbine. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - What do you mean by ‘Clausius inequality’? | 5M | CO-2 | BL-2 |
|  | - Derive an expression for the efficiency of the reversible heat engine. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6. | Give the Kelvin-Planck and Clausius statements of the second law and prove that violation of one law leads to the violation of another law and vice versa. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | a) Explain the following terms relating to steam formation: (i) Sensible heat of water, (ii) Latent heat of steam, | 5M | CO-2 | BL-2 |
|  | b) Determine the mass of 0.15 m3 of wet steam at a pressure of 4 bar and dryness fraction 0.8. Also calculate the heat of 1 m3 of steam. At 4 bar: vg = 0.462 m3/kg, hf = 604.7 kJ/kg, hfg = 2133 kJ/kg | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 8. | Derive the Maxwell relations and explain their importance in thermodynamics | 10M | CO-2 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | a) Develop an expression for thermal efficiency of air standard diesel cycle. | 5M | CO-3 | BL-3 |
|  | b) An air standard diesel cycle has a compression ratio of 17. The Pressure at the beginning of compression stroke is 1bar and the temperature is 230C. The maximum temperature is 14300C. Determine the thermal efficiency and the mean effective pressure for this cycle. Take ɼ = 1.4. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 10. | Define availability function and find the relationship between availability function and change in availability. | 10M | CO- | BL-2 |

***