| Subject Code 22PC1ME204 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January/February 2024

KINEMATICS OF MACHINERY

(ME)

Time: 3 hours                                                                                 		              Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | What is redundant link in a mechanism? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Describe the concept of Coriolis acceleration. | 2M | CO-2 | BL-2 |
|  | Write the conditions of correct steering. | 2M | CO-3 | BL-1 |
|  | What are the different types of motion with which a follower can move? | 2M | CO-4 | BL-1 |
|  | Define (i) ‘Law of Gearing’ and (ii) Velocity of Sliding’ | 2M | CO-5 | BL-1 |

PART- B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Explain the term kinematic link. Give the classification of kinematic link. | 5M | CO-1 | BL-2 |
|  | - Sketch and explain any two inversions of a double slider crank chain. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Define Grashof’s law. State how is it helpful in classifying the four-link mechanisms into different types | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | Explain the procedure to construct Klin’s construction to determine the velocity and acceleration of slider-crank mechanism. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4. | Locate all the instantaneous centres of the slider crank mechanism as shown in Fig. 6.12. The lengths of crank OB and connecting rod AB are 100 mm and 400 mm respectively. If the crank rotates clockwise with an angular velocity of 10 rad/s, find: 1. Velocity of the slider A, and 2. Angular velocity of the connecting rod AB. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - Enumerate straight line mechanisms. Why they are classified into exact and approximate straight line mechanisms. | 5M | CO-3 | BL-2 |
|  | - What is Hook’s joint? Where is it used? | 5M | CO-3 | BL-1 |
| OR |  |  |  |  |
| 6. | Give a neat sketch of the straight line motion ‘Hart mechanism.’ Prove that it produces an exact straight line motion. | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | - Draw the displacement, velocity and acceleration diagrams for a follower when it moves with simple harmonic motion. | 5M | CO-4 | BL-2 |
|  | - Derive the expression for velocity and acceleration during outstroke and return stroke of the follower. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | A cam operating a knife-edged follower has the following data : (a) Follower moves outwards through 40 mm during 60° of cam rotation. (b) Follower dwells for the next 45°. (c) Follower returns to its original position during next 90°. (d) Follower dwells for the rest of the rotation. The displacement of the follower is to take place with simple harmonic motion during both the outward and return strokes. The least radius of the cam is 50 mm. Draw the profile of the cam when 1. the axis of the follower passes through the cam axis, and 2. the axis of the follower is offset 20 mm towards right from the cam axis. If the cam rotates at 300 r.p.m., determine maximum velocity and acceleration of the follower during the outward stroke and the return stroke. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | - State and prove the law of gearing. Show that involute profile satisfies the conditions for correct gearing. | 5M | CO-5 | BL-2 |
|  | - Derive an expression for the length of the arc of contact in a pair of meshed spur gears. | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10. | Two parallel shafts are to be connected by spur gearing. The approximate distance between the shafts is 600 mm. If one shaft runs at 120 r.p.m. and the other at 360 r.p.m., find the number of teeth on each wheel, if the module is 8 mm. Also determine the exact distance apart of the shafts. | 10M | CO-5 | BL-3 |

***