| Subject Code 22PC1ME202 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January/February 2024

MACHINE TOOLS AND METROLOGY

(ME)

Time: 3 hours                                                                                 		              Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Distinguish between orthogonal and oblique cutting. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Explain the machining operations which can be done on reciprocating machine tools. | 2M | CO-2 | BL-1 |
|  | Elucidate the examples of products which are outcomes of broaching operation. | 2M | CO-3 | BL-2 |
|  | Write a brief note on slip gauges. | 2M | CO-4 | BL-1 |
|  | Define surface roughness and express the empirical relation to find the same. | 2M | CO-4 | BL-2 |

PART- B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) Briefly discuss about Geometry of single point cutting tool? Also, explain the following i) rake angle ii) Clearance angle iii) cutting angle iv) lip angle, with neat sketch. | 6M | CO-1 | BL-2 |
|  | b) Explain in detail different types of cutting tool materials. | 4M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Explain the process of chip formation during machining with neat sketches. Also discuss the types of chips. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | a) Explain any six types of operations which can be performed on a center lathe machine. | 6M | CO-2 | BL-2 |
|  | b) Distinguish between turret and capstan lathe machines. | 4M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | Sketch and explain a slotter machine with different types of operations. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | a) Write a note on milling operations. | 4M | CO-3 | BL-2 |
|  | b) What are the different hole making operations? Explain any three operations. | 6M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | Discuss various types of superfinishing operations. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | a) Distinguish between unilateral and bilateral tolerance systems. | 5M | CO-4 | BL-2 |
|  | b) Elucidate the significance of no-go gauges. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | Discuss the process of angular measurement using sine bar and spirit level. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | - Explain the process of measurement of surface finish. | 5M | CO-4 | BL-2 |
|  | - Discuss the functionality of tool maker’s microscope. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10. | Illustrate the procedure to measure effective diameter, angle of thread and thread pitch of a screw thread. | 10M | CO-4 | BL-3 |

***