| Subject Code 22PC1AE204 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January/February 2024

MECHANICS OF MATERIALS

(AE)

Time: 3 hours                                                                                 		              Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2 =10M

|  | What is principle of superposition? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define beam and list the different types of beams | 2M | CO-3 | BL-1 |
|  | Give the equation of polar modulus for solid circular shaft and a hollow shaft. | 2M | CO-1 | BL-1 |
|  | Define principal planes and principal stresses | 2M | CO-2 | BL-1 |
|  | State the assumptions made in Euler’s formula. | 2M | CO-3 | BL-1 |

PART- B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Illustrate the stress – strain diagram for mild steel with neat sketch and discuss salient points. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | A steel rod of 3 cm diameter and 5 m long is connected to two grips and the rod is maintained at a temperature of 95°C. Determine the stress and pull exerted when the temperature falls to 30° C, if - the ends do not yield, and - the ends yield by 0.12 cm. Take E=2x 105 MN/m2 and α = 12x 10-6/°C. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3. | A simply supported beam of length 6 m, carries point load of 3 kN and 6 kN at distances of 2 m and 4 m from the left end. Draw the shear force and bending moment diagrams for the beam. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 4. | A solid shaft transmits 200kW of power at 80rpm. Determine the diameter of the shaft if the shear stress is not to exceed 75MPa. If this shaft is replaced by a hollow shaft whose internal diameter is 0.6 of the external diameter while the length, material and the maximum shear stress are the same, find the percentage saving in weight. | 10M | CO-3 | BL-4 |
| UNIT-III |  |  |  |  |
| 5. | State the assumption made in the theory of simple bending equation and derive the simple bending equation. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | The cross section of the beam is shown. This beam is subjected to a shear force of 40kN. Determine the maximum shear stress and also sketch the shear stress distribution across the c/s of the beam. | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | At a point in a strained material the normal stresses are 100N/mm2 (tensile) and 60 N/mm2 (compressive). Determine the principle stresses and maximum shear stress by using Mohr’s circle method. | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
| 8. | An element in plane stress is subjected to stresses σx = -50 MPa, σy = 40 MPa, and τxy = -30 MPa. Determine the principal stresses, maximum shear stresses and show them on a sketch of a properly oriented element | 10M | CO-2 | BL-4 |
| UNIT-V |  |  |  |  |
| 9. | A beam 6 m long, simply supported at its ends, is carrying a point load of 50 kN at its centre. The moment of inertia of the beam is 78 x 106 mm4. If E for the material of the beam = 2.1 X 105 N/mm2. Calculate deflection at the center of the beam and slope at the supports by using Moment area method. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 10. | Calculate the safe compressive load on a hollow cast iron column of 150 mm external diameter, 100 mm internal diameter and 10 m length. Use Euler’s formula with E = 95 GN/m2. - One end rigidly fixed and the other hinged. - Both the ends hinged. | 10M | CO-3 | BL-4 |

***