| Subject Code 22PC1ME201 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January /February 2024

MECHANICS OF SOLIDS

(ME)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART – A

5X2=10M

|  | Define the Factor of Safety and write its importance. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Discuss the shear force and bending moment diagrams | 2M | CO-2 | BL-1 |
|  | Write the equation for deflection in a simply supported beam subjected Point Load at the mid span. | 2M | CO-3 | BL-2 |
|  | What is principal plane and principal stress? | 2M | CO-4 | BL-1 |
|  | When is Macaulay’s method useful? | 2M | CO-5 | BL-2 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Explain briefly about the stress – strain diagram for mild steel with neat sketch. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | A member ABCD is subjected to point loads P1, P2, P3 and P4 as shown in figure below. Calculate the force P2 necessary for equilibrium, if P1 = 45 kN, P3=450 kN and P4=130 kN. Determine the total elongation of the member, assuming the modulus of elasticity to be 2.1 x 105 N/mm2. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3 | Draw shear force and bending moment diagram for the beam shown in figure below. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | A Simply supported beam of 10 m long is subjected to a UDL of 20 kN/m at a distance of 4 m from the left support and also carries a another UDL of 10 kN/m at a distance of 4 m from the right support. Draw the shear force and bending moment diagrams for the beam. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | A timber beam of rectangular section carries a load of 2 kN at mid-span. The beam is simply supported over a span of 3.6 m. If the depth of section is to be twice the breadth, and the bending stress is not to exceed 9 N/mm2, determine the cross-sectional dimensions. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6 | A beam of I-section shown in the figure is subjected to a shear force of 40kN. Determine the shear stress distribution for the given beam section. | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7 | A point in a strained material is subjected to stresses shown in Fig. Using Mohr's circle method, determine the normal and tangential stresses across the oblique plane. Check the answer analytically. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 8 | At a point in a strained material the principal stresses are 100 N/mm2 (tensile) and 60 N/mm2 (compressive). Determine the normal stress, shear stress and resultant stress on a plane inclined at 50° to the axis of major principal stress. Also determine the maximum shear stress at the point. | 10M | CO-4 | BL-4 |
| UNIT-V |  |  |  |  |
| 9 | A beam of length 6m is simply supported at ends and carries two-point loads of 48 kN and 40 kN at distance of 1m and 3m respectively from the left support. Using Macauley’s method find (i) deflection under each load (ii) maximum deflection (iii) the point at which maximum deflection occurs. Given, E = 2 x 105 N/mm2 & I = 85 x 106 mm4 | 10M | CO-5 | BL-4 |
| OR |  |  |  |  |
| 10 | A beam 6m long, simply supported at its ends, is carrying a point load of 50 kN at its centre. The moment of inertia of the beam is 78 x 106 mm4. If E for the material of the beam = 2.1 X 105 N/mm2. Calculate deflection at the centre of the beam and slope at the supports. | 10M | CO-5 | BL-4 |

***