| Subject Code 22PC1CE201 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January/February 2024

STRENGTH OF MATERIALS-I

(CE)

Time: 3Hrs											      Max.Marks:60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART – A

5X2=10M

|  | Define Poisson’s ratio. | 2M | CO1 | BL1 |
| --- | --- | --- | --- | --- |
|  | What is Composite bar? | 2M | CO1 | BL1 |
|  | What is meant by point of contraflexure? | 2M | CO2 | BL1 |
|  | Write the equation for simple bending theory. | 2M | CO3 | BL1 |
|  | Draw the shear stress distribution for symmetric I-section? | 2M | CO4 | BL2 |

PART- B

5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Draw the stress-strain plot for ductile and brittle materials. | 4M | CO-1 | BL-2 |
|  | - Find the total extension of the bar as shown in Figure. Take E=200 Gpa. | 6M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 2. | A steel plate 300 mm long, 60 mm wide and 30 mm deep is acted upon by the forces shown in Figure. Determine the changes in volume from first principles. Take E=2x105 N/mm2 and Poisson’s ratio = 0.30. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3 | - Derive the strain energy equation for axially loaded member. | 5M | CO-1 | BL-2 |
|  | b) A steel rod of 5 cm diameter and 4 m long is connected to two grips and the rod is maintained at a temperature of 1000C. Determine the stress and pull exerted when the temperature fall to 300C, if (i) the ends do not yield and (ii) the ends yield by 0.12cm. Take E=2*105 N/mm2 and α=12*10-6/0C. | 5M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 4 | The compound tube consists of a steel and brass tubes. The steel tube having 150 mm internal diameter and 180 mm external diameter and an outer brass tube having 180 mm internal diameter and 200 mm external diameter. The tubes are of the same length. The compound tube carries an axial load of 40 kN. Find the stresses and load carried by each tube and the amount of shortening if length of tube is 140 mm. Take ES=2*105 N/mm2 and EB=1*105 N/mm2. | 10M | CO-1 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | Draw SFD and BMD for the beam shown in Figure. Find out salient features if any. | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
| 6 | Draw SFD and BMD for the beam shown in Figure and also find location and magnitude of maximum bending moment. | 10M | CO-2 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7 | The SSB of span 4 m has a cross section as shown in Figure. If the permissible stress of the material of beam is 140 N/mm2. To determine (i) the maximum UDL it can carry, (ii) the maximum point load it can carry at the center of span, and (iii) the maximum point load if can carry at a distance of 1 m from left support. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 8 | State the assumptions and derive the equation of pure bending equation. | 10M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | Draw the shear stress variation diagram for the T-section shown in Figure, if it is subjected to a shear force of 100 kN. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10 | Draw the shear stress variation diagram for the I-section shown in Figure, if it is subjected to a shear force of 150 kN. | 10M | CO-4 | BL-3 |

***