| Subject Code 22PC1ME203 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular Examinations, January/February 2024

THERMODYNAMICS

(ME)

Time: 3 hours                                                                                 		              Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2 = 10M

|  | What is meant by system &surrounding? | 2M | CO-1,3 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define availability | 2M | CO-1,3 | BL-1 |
|  | State the Clausius statement of second law of Thermodynamics | 2M | CO-1,3 | BL-2 |
|  | Define dryness fraction ? | 2M | CO-2,3 | BL-1 |
|  | Explain Equation of State | 2M | CO-2,3 | BL-2 |

PART- B

5X10=50M

| UNIT-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - A fluid at a pressure of 3 bar and with specific volume of 0.18 m3 /kg contained in a cylinder behind a piston expands reversibly to a pressure of 0.6 bar according to a law , p=c/v2 where c is a constant .Calculate the work done by the fluid on the piston. | 5M | CO-1,3 | BL-3 |
|  | - What is quasi static process? Explain in detail? | 5M | CO-1,3 | BL-1 |
| OR |  |  |  |  |  |
| 2. | Define Property and state of a substance and prove that Energy is property of the system. | 10M | CO-1,3 | BL-1 |
| UNIT-II |  |  |  |  |  |
| 3. | - Write short notes on following associated with S.F.E.E. (i) Nozzle (ii) Throttle Valve (iii) Turbine | 5M | CO-1,3 | BL-2 |
|  | - Explain the concepts of availability and irreversibility. | 5M | CO-1,3 | BL-2 |
| OR |  |  |  |  |  |
| 4. | Two streams of air, one at 1 bar, 27 deg.C and velocity of 30 m/s and the other at bar, 227 deg C and velocity of 50 m/s mix in equal proportion in a chamber from which heat at the rate of 100 kJ/kg is removed. The mixture is then passed through an adiabatic nozzle. Find the velocity of the stream issuing out of the nozzle. The temperature of the air leaving the nozzle is 27 deg C and its Cp = 1.005 kJ/kgK . | 10M | CO-1,3 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5. | - Define COP and Prove that COP of heat pump is 1+ COP of refrigeration based on second law of thermodynamics | 5M | CO-2,3 | BL-2 |
|  | - Discuss the concept of principle of entropy increases. | 5M | CO-2,3 | BL-2 |
| OR |  |  |  |  |  |
| 6. | Two Carnot engines A and B are connected in series between two thermal reservoirs maintained at 1000 K and 100 K respectively. Engine A receives 1680 kJ of heat from the high-temperature reservoir and rejects heat to the Carnot engine B. Engine B takes in heat rejected by engine A and rejects heat to the low-temperature reservoir. If engines A and B have equal thermal efficiencies, determine - The heat rejected by engine B - The temperature at which heat is rejected by engine, A - The work done during the process by engines, A and B respectively. - If engines A and B deliver equal work, determine - The amount of heat taken in by engine B - The efficiencies of engines A and B | 10M | CO-2,3 | BL-4 |
| UNIT-IV |  |  |  |  |  |
| 7. | - With neat sketch explain the formations of stem using T-h diagram. | 5M | CO-2,3 | BL-1 |
|  | - Steam at a pressure of 6 bar and with 41.2 degree C degree of superheat is expanded adiabatically and reversibly in a cylinder to 1 bar. Using steam tables, Calculate the work done per kg of steam during | 5M | CO-2,3 | BL-3 |
| OR |  |  |  |  |  |
| 8. | An SI engine operating on Otto cycle, has its inlet parameters as 323 K and 1 bar, while it develops a maximum pressure of 25 bar. Assuming Bore as 250 mm, Stroke as 375 mm and clearance volume as 0.00263 m3 , determine (i) air standard efficiency and (ii) mean effective pressure of the cycle. | 10M | CO-2,3 | BL-4 |
| UNIT-V |  |  |  |  |  |
| 9. | - Explain the Dalon’s law of partial pressures for mixture of gases. | 5M | CO-1,3 | BL-2 |
|  | - Derive the expressions for the internal energy and specific heats for mixtures of ideal gases. | 5M | CO-1,3 | BL-2 |
| OR |  |  |  |  |  |
| 10. | A mixture of ideal gases consists of 7kg of and 2kg of at a pressure of 4bar and a temperature of 27°C. Determine: (i) Mole fraction of each constituent,(ii) Equivalent molecular weight of the mixture,(iii)Equivalent gas constant of the mixture,(iv)The partial pressure and partial volumes,(v) The volume and density of the mixture. | 10M | CO-1,3 | BL-4 |

***