| Subject Code 22PC1EC210 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular Examinations, July 2024

ANALOG ELECTRONIC CIRCUITS

(EEE)

Time: 3 hours                                                                                 		               Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Mention important characteristics of CE amplifier. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What are the advantages of push pull amplifiers? | 2M | CO-2 | BL-1 |
|  | Write the differences between while RC network working as a differentiator and Integrator. | 2M | CO-3 | BL-1 |
|  | Write the properties of ideal op-amp. | 2M | CO-4 | BL-1 |
|  | Draw the pin diagram of 555 Timer. | 2M | CO-5 | BL-1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Derive the expression for the CE short circuit current gain Ai as a function of frequency using Hybrid - π model and draw the frequency response. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Explain the effect of coupling capacitor on frequency response of BJT amplifier with neat diagram. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | What is harmonics distortion in power amplifier? Discuss the operation of a class B push pull power amplifier and derive its maximum power conversion efficiency. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4. | Discuss in detail about Darlington amplifier with circuit diagram, expression and write it’s advantages and disadvantages. | 10M | CO-1 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | a) Describe the operation of two-level slicer with the help of circuit diagram and wave forms. | 5M | CO-3 | BL-2 |
|  | b) Design a clamper circuit using diode to obtain sine wave output with its negative peak clamped to +2.6V. (Assume diode drop as 0.6V). | 5M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 6. | Explain the response of high pass RC circuit when it is applied with - Step input b) pulse input | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | a) Design an adder circuit using an op-amp to get the output expressions as Vout= -(V1+5V2+25V3), where V1, V2 and V3 are the inputs. Given that Rf=50 kΩ. | 5M | CO-4 | BL-3 |
|  | b) Explain the operation of an op-amp comparator with circuit diagram and waveforms | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | With a neat circuit diagram, explain the operation of an Instrumentation amplifier and derive an expression for its voltage gain. What are its advantages? | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | - Construct the R-2R ladder network DAC and derive the expression. | 5M | CO-5 | BL-3 |
|  | - Discuss the importance of Parallel comparator ADC with design. | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10. | With the help of a neat diagram explain the operation of a monostable multivibrator using 555 IC. | 10M | CO-5 | BL-3 |

***