| Subject Code 22PC1EE205 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular Examinations, July 2024

CONTROL SYSTEMS

(Common to ECE and EIE)

Time: 3 hours                                                                                 		               Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					 5X2=10M

| a) | What do you mean by closed loop system? Give the examples of closed loop system. | 2M | CO2 | BL-1 |
| --- | --- | --- | --- | --- |
| b) | Convert the given signal flow graph into Block diagram. | 2M | CO1 | BL-3 |
| c) | What is called steady state stability. | 2M | CO2 | BL-1 |
| d) | Define Delay time and Rise time. | 2M | CO4 | BL-1 |
| e) | List and prove the properties of State Transition Matrix. | 2M | CO3 | BL-1 |

PART-B					 5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | What are the three basic elements in the Translational mechanical system? Find transfer function for the given system. | 10M | CO1 | BL-3 |
| OR |  |  |  |  |
| 2. | State the block diagram reduction rules. Find the closed loop transfer function of the system for the given block diagram. | 10M | CO3 | BL-4 |
| UNIT-II |  |  |  |  |
| 3. | A unity feedback control system has a loop transfer function, G(s) =10/s(s+2). Find the rise time, percentage overshoot, peak time and settling time for unit step input | 10M | CO1 | BL-3 |
| OR |  |  |  |  |
| 4. | A feedback Control Systems has an open loop transfer function: GSHs=Ks(s+3)(s2+2s+2) Find the root locus as K is varied from 0 to ∞ | 10M | CO2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | Use Bode plot to determine the gain crossover frequency, phase crossover frequency, gain margin and phase margin of a unity feedback system with open loop transfer function. Gs=10s(1+0.1s)(1+0.4s) . | 10M | CO2 | BL-3 |
| OR |  |  |  |  |
| 6. | Explain the following terms. - Resonant frequency - Resonant peak. - Bandwidth. - Gain margin - Phase margin. | 10M | CO2 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | What is the purpose of Controller? Explain Proportional, Integral and Derivative controllers. | 10M | CO4 | BL-2 |
| OR |  |  |  |  |
| 8. | A unity feedback system has an open loop transfer function GS=KS3+S(10+S). Design a suitable lag compensation so that phase margin is 450 and velocity error constant, Kv ≤15. | 10M | CO4 | BL-4 |
| UNIT-V |  |  |  |  |
| 9. | Determine the state transition matrix for the system X = Ax, where A=-2010-1120-1 | 10M | CO3 | BL-3 |
| OR |  |  |  |  |
| 10. | A linear time invariant system is characterized by the homogeneous state equation. Find the time response of the system. Assume the initial state vector. | 10M | CO3 | BL-3 |

***