| Subject Code 22PC1DS202 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular Examinations, July 2024

DIGITAL LOGIC DESIGN AND COMPUTER ORGANIZATION

(Common to CSE-DS, CSE-AIML and AIDS)

Time: 3 hours                                                                                 		               Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					 5X2=10M

|  | Convert 1011011 to gray code. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Differentiate between combinational circuit sequential circuits. | 2M | CO-2 | BL-1 |
|  | Illustrate about fixed and floating-point representations? | 2M | CO-3 | BL-1 |
|  | Define pipelining and throughput? | 2M | CO-4 | BL-1 |
|  | List the characteristics of peripheral devices? | 2M | CO-5 | BL-1 |

PART-B					 5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - Convert (125.45)10= (-----)2; (111001001.0111)2= (-----)10; (3245)8= (-----)10? | 5M | CO-1 | BL-4 |
|  | - Simplify F= ∑ (1, 4, 5, 6, 9, 10, 12, 15) using four variable K Map and obtain the expression in SOP and POS forms? | 5M | CO-1 | BL-4 |
| OR |  |  |  |  |  |
| 2. | Implement NOT, AND, OR, NAND, EXOR, EXNOR gates using NOR gates? | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3. | - Design full Adder using half Adders? | 5M | CO-2 | BL-3 |
|  | - Convert JK to T and D flipflops? | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 4. | Design a Mod 8 synchronous counter using JK flipflop and also explain the effect of unused states? | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |  |
| 5. | - Analyse in detail about the different types of addressing modes with an example for each? | 5M | CO-3 | BL-2 |
|  | - Elaborate in detail about the concept of instruction execution cycle? | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 6. | With the help of an example discuss in detail about the operation of a booth multiplier? | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |  |
| 7. | - Elaborate in detail about pipeline hazards? | 5M | CO-4 | BL-2 |
|  | - Illustrate in detail about the basic concepts of pipelining? | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 8. | Explain micro program sequencer for a control memory with a neat block diagram. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9. | - Illustrate in detail about the concept of DMA? | 5M | CO-5 | BL-2 |
|  | - Elaborate in detail about the concept of I/O driven interface? | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |  |
| 10. | Discuss in detail about the memory hierarchy and its sub classifications? | 10M | CO-5 | BL-2 |

***