| Subject Code 22PC1ME206 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular Examinations, July 2024

DYNAMICS OF MACHINERY

(ME)

Time: 3 hours                                                                                 		                Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Differentiate static force analysis and dynamic force analysis. | 2M | CO-2 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Write and explain the equation for calculating the change in fluctuation of energy. | 2M | CO-1 | BL-2 |
|  | What is the function of a clutch? | 2M | CO-2 | BL-2 |
|  | What do you mean by dynamic balancing of a system? | 2M | CO-3 | BL-1 |
|  | What are various types of vibrations? | 2M | CO-4 | BL-2 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | An aero plane makes a complete half circle of 40m radius, towards left, when flying at 200 km/hr. The rotary engine and the propeller of the plane have a mass of 300 kg with a radius of gyration of 250 mm. The engine runs at 2000 r.p.m. clockwise, when viewed from the rear. Determine the gyroscopic couple on the aircraft and its effect on the aeroplane. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 2. | Derive Freudentstein’s equation with the help of four bar mechanism. | 10M | CO-2 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | The turning moment diagram for a petrol engine is drawn to the following scales : Turning moment, 1 mm = 5 N-m ; crank angle, 1 mm = 1°. The turning moment diagram repeats itself at every half revolution of the engine and the areas above and below the mean turning moment line taken in order are 295, 685, 40, 340, 960, 270 mm2. The rotating parts are equivalent to a mass of 36 kg at a radius of gyration of 150 mm. Determine the coefficient of fluctuation of speed when the engine runs at 1800 r.p.m. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 4. | Draw the controlling force diagram for a Hartnell governor and discuss about stable, unstable and isochronous governor. | 10M | CO-1 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | Determine the maximum, minimum and average pressure in plate clutch When the axial force is 4 kN. The inside radius of the contact surface is 50 mm and the outside radius is 100 mm. Assume uniform wear. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6. | A single block brake is shown in figure. The diameter of the drum is 250 mm and the angle of contact is 90°. If the operating force of 700 N is applied at the end of a lever and the coefficient of friction between the drum and the lining is 0.35, determine the torque that may be transmitted by the block brake. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | Explain clearly the terms ‘static balancing’ and ‘dynamic balancing’. State the necessary conditions to achieve them. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 8. | Four masses A, B, C and D are attached to a shaft and revolve in the same plane. The masses are 5 kg, 4 kg, 8 kg and 6 kg respectively and their radii of rotations are 30 mm, 40 mm, 50 mm and 25 mm. The angular position of the masses B, C and D are 60°, 150° and 240° from the mass A. Find the magnitude and position of the balancing mass at a radius of 100 mm. | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |
| 9. | Calculate the whirling speed of a shaft 50 mm diameter and 0.5 m long carrying a mass of 1 kg at its mid-point. The density of the shaft material is 1Kg/m3 and Young’s modulus is 200 GPa. Assume the shaft to be freely supported. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 10. | a) Find the natural frequency of a block of mass 50 kg placed at the mid-span of a simply supported beam of length 5m. Moment of inertia and Young’s modulus of the beam are 250 mm4 and 2.1×105 N/mm2, respectively. b) A vibrating system is defined by the parameters as given below: mass, m=10 kg, spring stiffness, k=100 kN/m, damping coefficient, C=50 N/m/s, Determine the critical damping co-efficient. | 5M 5M | CO-4 CO-4 | BL-4 BL-4 |

***