| Subject Code 22PC1EE203 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular Examinations, July 2024

ELECTRICAL MACHINES-II

(EEE)

Time: 3 hours                                                                                 		      	    Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | What are the advantages of skewing of rotor slots in squirrel cage induction motor. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define Cogging effect. | 2M | CO-2 | BL-1 |
|  | Write a short note on shaded pole Induction Motor. | 2M | CO-2 | BL-2 |
|  | What are the different methods of finding voltage regulations of 3 phase Alternator? | 2M | CO-3 | BL-1 |
|  | How a synchronous motor can be used as synchronous condenser. | 2M | CO-2 | BL-1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) Explain the principle of operation of 3 phase Induction motor. | 5M | CO-2 | BL-2 |
|  | b) A three-phase induction motor is wound for 4 poles and is supplied from 50 HZ System. Calculate (a)synchronous speed (b) speed of the motor when slip is 4% and (c) Rotor current frequency when the motor runs at 600rpm? | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 2. | Derive the following. - Torque equation of an induction motor. - Condition for Maximum Torque under running condition. | 10M | CO-3 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | a) Draw the power flow diagram and derive the relation of rotor input:rotor cu losses: rotor power developed= 1:S: (1-S). | 5M | CO-3 | BL-3 |
|  | b) With the help of circuit diagram explain Auto Transformer starting of induction motor. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4. | Explain the procedure to draw the circle diagram of 3-phase Induction motor and find the performance characteristics. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | - Explain Double-Field Revolving Theory of single-phase induction motor. | 5M | CO-2 | BL-2 |
|  | - Explain the split phase and capacitor start methods of induction motor. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6. | Discuss no load and blocked rotor test of single-phase Induction motor and draw the equivalent circuit through which derive the relevant equations. | 10M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | - Derive the EMF equation of an Alternator. | 5M | CO-3 | BL-3 |
|  | - Explain Armature reaction in an Alternator. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 8. | Explain the procedural steps to find Xd and Xq from Slip test. | 10M | CO-2 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | a) Explain any two starting methods of 3 phase synchronous motor. | 5M | CO-2 | BL-2 |
|  | b) Discuss the V & inverted V curves of synchronous motor. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 10. | Draw the phasor diagram of synchronous motor for different power factors. | 10M | CO-2 | BL-2 |

***