| Subject Code 22PC1EI203 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular Examinations, July 2024

ELECTRONIC DEVICES AND CIRCUITS-II

(EIE)

Time: 3 hours                                                                                 		      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Write the difference between clipping and clamping. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Draw the h-parameter model of the single stage CE Amplifier. | 2M | CO-2 | BL-2 |
|  | Justify the name of Complementary symmetry power amplifier. | 2M | CO-3 | BL-2 |
|  | Differentiate between LCD and LED. | 2M | CO-4 | BL-2 |
|  | List the different methods followed to take SCR from its conducting state to the non-conducting state. | 2M | CO-4 | BL-2 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - Prove that an RC circuit behaves as a reasonably good integrator if RC >15T, where T is the period of an input Emsinwt | 5M | CO-1 | BL-2 |
|  | - Design a circuit which clips the positive peak above 3V for an input of sinusoidal signal with 10VP-P and draw its waveform neatly. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
| 2. | - Analyze the response of a Low pass RC circuit when a Step input is applied and also find the general expression for the voltage across the capacitor. | 5M | CO-1 | BL-4 |
|  | - Define rise time tr of an RC Low pass circuit and show that tr = 2.2 RC. Also find the relation between rise time and upper 3-dB frequency. | 5M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3. | Calculate Ai, Av, Ais, Avs, Ri and Ro for a Darlington amplifier, if circuit parameters are Rs=1KΩ, RC1=15KΩ, RE1=100Ω, RC2=4KΩ, RE2=330Ω, R1 and R2 for 2 stages are 100KΩ and 20KΩ respectively. Assume hie=1.2KΩ, hfe=50, hre=2.5x10-4 and hoe=25x10-6 A/V. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 4. | Develop expressions for Current gain, Voltage gain, Input Resistance and Output Resistance of two stage RC coupled amplifier. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |  |
| 5. | - With the help of a neat diagram, explain the operation of a transformer coupled Class-B power amplifier. | 6M | CO-3 | BL-2 |
|  | - What is heat sink and mention its role with respect to power amplifiers. | 4M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 6. | - Derive an expression for the efficiency of series fed Class-A power amplifiers. | 5M | CO-3 | BL-3 |
|  | - Design a series fed Class-A power amplifier to achieve maximum output power to a 10Ω load. The two transistors have the following rating. Pd(max) = 4W, VCE (max) = 40V, Ic(max) = 1A, =50. Assume data wherever necessary. | 5M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |  |
| 7. | - Discuss the principle of operation and VI characteristics of UJT. | 5M | CO-4 | BL-2 |
|  | - Draw and explain the characteristics of Schottky barrier diode. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 8. | Explain the principle of operation and characteristics of Tunnel diode with the help of energy band diagrams. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9. | - Illustrate and describe the operation of a Schockley Diode. | 5M | CO-4 | BL-2 |
|  | - Explain the structural functioning and characteristics of the DIAC. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 10. | Illustrate and describe the characteristics of a SCR and explain the two-transistor analogy of a SCR. | 10M | CO-4 | BL-2 |

***