| Subject Code 22PC1AE208 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular Examinations, July 2024

FLUID MECHANICS AND HEAT TRANSFER

(AE)

Time: 3 hours                                                                                 		               Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Define surface tension and viscosity. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List out the applications of Bernoulli‘s equation. | 2M | CO-2 | BL-2 |
|  | State Fourier’s law of heat conduction. | 2M | CO-3 | BL-2 |
|  | Draw velocity profile for fully developed flow in circular tubes. | 2M | CO-3 | BL-2 |
|  | What is a gray body? | 2M | CO-4 | BL-1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Define manometers and mention its types? | 5M | CO-1 | BL-1 |
|  | - What are stream function and velocity potential functions and write their mathematical expression. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Determine the components of acceleration at a point (3, 1, 2) when the ﬂow is described by V = (y2+z2 )i + ( x2+ z 2) j + (x2 + y2) k. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | a) Derive an expression for Darcy – Weisbach equation. b) The water is ﬂowing through a tapering pipe having diameters 0.25 m and 0.125 m at sections 1 and 2, respectively. The discharge through the pipe is 40 litres per second. The section 1 is 5 m above the datum and section 2 is 3 m above the datum. If the pressure at section 1 is 0.4 MPa, then determine the intensity of pressure at section 2 | 5M 5M | CO-2 CO-2 | BL-3 BL-3 |
| OR |  |  |  |  |
| 4. | Derive the Bernoulli’s equation from the first principles. | 10M | CO-2 | BL- 2 |
| UNIT-III |  |  |  |  |
| 5. | Derive the general conduction equation for 3-D in cylindrical co-ordinate system. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | An exterior wall of a house may be approximated by a 0.1 m layer of common brick (K=0.48 W/m0C) followed by a 0.04m layer of gypsum plaster (K=0.48 W/m0C). What thickness of loosely packed rock wool insulation (K=0.065 W/m0C) should be added to reduce the heat loss through the wall by 80 percent? | 10M | CO-3 | BL- 3 |
| UNIT-IV |  |  |  |  |
| 7. | Explain about the formation of hydrodynamic and thermal boundary layers, non-dimensional correlation for forced convection heat transfer. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 8. | Air flows over a flat plate at a constant velocity of 5m/sec and at 2000C. The plate is 15 mm wide and maintained at 120 0C. Find the hydrodynamic boundary layer thickness, thermal boundary layer thickness and heat transfer rate along the length of the plate of 50cm. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | Derive the expression for the LTD for a parallel flow heat exchanger? | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10. | Two large parallel planes having emissivity 0.3 and 0.4 are maintained at a temperature 800 0C and 500 0C respectively. A radiation shield of emissivity 0.05 on both sides is placed in between. Determine the i) rate of heat transfer per unit area if shield is not there ii) heat transfer with the shield iii) the temperature of the shield. | 10M | CO-4 | BL-3 |

***