| Subject Code 22PC1ME207 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular Examinations, July 2024

FLUID MECHANICS AND HYDRAULIC MACHINES

(ME)

Time: 3 hours                                                                                 		               Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | State Pascal’s law and give some examples where this principle is applied. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What are the assumptions taken in deriving the Bernoulli’s equation? | 2M | CO-2 | BL-2 |
|  | What is the purpose of drawing velocity triangles | 2M | CO-3 | BL-2 |
|  | Define specific speed of turbine and its significance | 2M | CO-4 | BL-1 |
|  | What is NPSH and what is its significance | 2M | CO-4 | BL-1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Calculate the capillary rise in a glass tube of 2.5mm diameter when immersed vertically in i) water and ii) mercury. Take surface tensions =0.0725 N/m for water and =0.52 N/m for mercury in contract with air. The specific gravity for mercury is given as 13.6 and angle of contact=130o. | 5M | CO-1 | BL-3 |
|  | - Explain briefly the working principle of U-tube differential manometer. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | - Define stream lines and streak lines and also explain how do they differ from each other for steady and unsteady flow. | 5M | CO-1 | BL-3 |
|  | - The velocity at a point is given by V=(4t+3x2+2y)I +(t2+2xy+3y2)j. Determine the local acceleration and total acceleration at a point (2,3) for t=2seconds. | 5M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - Derive an expression for the head loss due to sudden enlargement in pipe flow. | 5M | CO-2 | BL-2 |
|  | - A horizontal venturimeter with inlet diameter 20 cm and throat diameter 10 cm is used to measure the flow of oil of specific gravity 0.8. The discharge of oil through venturimeter is 60 litres/sec. Find the reading of the oil- mercury differential manometer. Take Cd = 0.98. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | Three pipes of 400mm, 200mm and 300mm diameter having lengths of 400m, 200m and 300m respectively. They are connected in series to make a compound pipe. The ends of this compound pipe are connected with two tanks whose difference of water level is 16m. If co-efficient of friction for these pipes is same and equal to 0.05. Determine the discharge through the compound pipe neglecting first the minor losses and including them. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - A 50 mm diameter jet having a velocity of 18 m/s impinges without shock on a flat plate inclined at an angle of 30o to the axis of the jet. If the plate is moving at 5 m/s in the direction of jet, make calculations for the normal force exerted on the plate, work done and the efficiency. | 5M | CO-3 | BL-3 |
|  | - Derive an equation for the impact of jet striking a symmetrical moving curved vane at the centre. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | A jet of water of diameter 75mm, having a velocity of 30 m/s strikes a curved vane which is moving with a velocity of 15 m/s in the direction of jet. The jet leaves the vane at an angle of 60o to the direction of motion of vane at outlet determine the force exerted by jet on the vane in the direction of motion, work done per second by the jet. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | - What is draft tube? Describe with neat sketches different types of draft tubes. | 5M | CO-4 | BL-2 |
|  | - Explain the factors which decide the choice for a particular hydraulic turbine for a hydro-power project. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | A reaction turbine works at 460 rpm under a head of 110 metres. Its diameter at inlet is 1150 mm and the flow area is 0.03 m2 . The angles made by the absolute velocity and relative velocity at inlet are respectively 18oand 50owith the tangential velocity. Determine (i) The volume flow rate (ii) The power developed (iii) The efficiency. Assume whirl at outlet to be zero. | 10M | CO-4 | BL-4 |
| UNIT-V |  |  |  |  |
| 9. | - Derive the specific speed of centrifugal pump | 5M | CO-4 | BL-2 |
|  | - Draw and discuss characteristic curves of a centrifugal pump. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10. | Describe the principle and working of a reciprocating pump with a neat sketch. | 10M | CO-4 | BL-2 |

***