| Subject Code 22PC1ME210 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular Examinations, July 2024

FLUID MECHANICS AND MACHINERY

(EEE)

Time: 3 hours                                                                                 		               Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					     5X2=10M

|  | Define the Density and specific weight of the fluid? write its units. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Write any two assumptions in derivation of Bernoulli’s equation? | 2M | CO-2 | BL-1 |
|  | What is meant by the impact of Jet? | 2M | CO-3 | BL-1 |
|  | Differentiate the impulse and reaction turbine. | 2M | CO-1 | BL-2 |
|  | Define the slip of the pump? Write the expression of % of slip. | 2M | CO-1 | BL-2 |

PART-B					   5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Explain the measurement of pressure using simple manometer with neat sketch. | 4M | CO-1 | BL-2 |
|  | - Derivate the continuity equation for One dimensional flow. | 6M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Water flows through a pipe AB 1.2 m diameter at 3 m/s and then passes through a pipe BC 1.5 m diameter. At C, the pipe branches. Branch CD is 0.8 m in diameter and carries one-third of the flow in AB. The flow velocity in branch CE is 2.5 m/s. Find the volume rate of flow in AB, the velocity in BC , the velocity in CD and the diameter of CE. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3. | Water is flowing through a pipe having diameters 20 cm and 10 cm at sections 1 and 2 respectively. The rate of flow through pipe is 35 litres/sec. the section 1 is 6 m above datum and section 2 is 4 m above datum. If the pressure at section 1 is 39.24 N/cm2, find he intensity of pressure at section 2. | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
| 4. | a) Explain the about the Reynold’s Experiment with neat sketch. | 5M | CO-2 | BL-2 |
|  | b) Explain how do you find the resultant force exerted by the fluid on the pipe bend. | 5M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | Discuss the hydro electric power plant with neat sketch. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | A jet of water of diameter 7.5 cm strikes a curved plate at its entre with a velocity of 20m/s. The curved plate is moving with a velocity of 8 m/s in the direction of the jet. The jet is deflected through an angle of 1650. Assuming the plate smooth find: - Force exerted on the plate in the direction of jet - Power of the jet - Efficiency of the jet | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | A Pelton wheel has a mean bucket speed of 10 m/s with a jet of water flowing at the rate of 700 lit/sec under a head of 30 m. The buckets deflect the jet through an angle of 1600. Calculate the power given by water to the runner and the hydraulic efficiency of the turbine. Take coefficient of velocity as 0.98 | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 8. | - Explain the working principle of Pelton turbine with a neat sketch. | 6M | CO-4 | BL-2 |
|  | - List out the types of draft tubes used in hydraulic turbines. | 4M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | Differentiate between single – stage and multistage centrifugal pumps. Describe multistage pump with i) impellers in parallel (ii) impellers in series. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10. | - Explain the working of single acting reciprocating pump with neat sketch. | 5M | CO-4 | BL-2 |
|  | - Define indicator diagram. Explain double acting reciprocating pump. | 5M | CO-4 | BL-4 |

***