| Subject Code 22ES1EE103 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular Examinations, July 2024

FUNDAMENTALS OF ELECTRICAL AND ELECTRONICS ENGINEERING

(ME)

Time: 3 hours                                                                                 		               Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					    5X2=10M

|  | Find the impedance of a series RLC circuit with R=5Ω, L= 400mH, C= 50μF supplied with a 50Hz A.C supply. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | List the types of DC generators. | 2M | CO-3 | BL-1 |
|  | Define efficiency & regulation of a transformer. | 2M | CO-2 | BL-1 |
|  | Draw the slip – torque characteristics of an induction motor. | 2M | CO-3 | BL-2 |
|  | Write the truth tables of AND, OR and NOT logic gates. | 2M | CO-4 | BL-1 |

PART-B					  5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - State and Explain KVL & KCL with an example. | 5M | CO-1 | BL-2 |
|  | b) For the circuit shown below, find (i) the current I (ii) the voltage across 30Ω resistor. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | a) Find the average, RMS values and form factor of a sinusoidal function. | 5M | CO-1 | BL-2 |
|  | b) Explain the voltage current relation in a series R-L-C circuit and also draw its phasor diagram. | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | a) Derive the E.M.F equation of a DC generator. | 5M | CO-3 | BL-3 |
|  | b) Explain the speed control methods of DC shunt motor with neat sketch. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 4. | a) Derive the Torque equation of a DC motor. | 5M | CO-3 | BL-3 |
|  | b) A D.C generator generates a voltage of 200V when run at 1000rpm. Find the voltage generated when the generator is run at 1500 rpm and the flux per pole is halved. | 5M | CO-3 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | Explain the operation of transformer and also derive it’s E.M.F. equation. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6. | a) What are the various losses occurring in a 1- ф transformer. On what factors do they depend? | 5M | CO-2 | BL-2 |
|  | b) With a neat circuit diagram explain the procedure of OC & SC tests on transformer. | 5M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | Explain the construction & working principle of an induction motor with a neat sketch. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 8. | a) Explain the principle of an alternator. | 5M | CO-3 | BL-2 |
|  | b) Discuss in detail about determination of regulation of alternator by synchronous impedance method. | 5M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | a) Explain the Operation and VI Characteristics of a PN Junction diode. | 5M | CO-4 | BL-2 |
|  | b) Draw the characteristics of a SCR and explain its working. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10. | Explain the working of BJT and also draw the input-output characteristics in CE configuration. | 10M | CO-4 | BL-2 |

***