| Subject Code 22PC1EE204 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular Examinations, July 2024

POWER SYSTEMS-I

(EEE)

Time: 3 hours                                                                                 		               Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Explain the function of Economizer in thermal power plant. | 2M | CO 1 | BL 2 |
| --- | --- | --- | --- | --- |
|  | Define GMR and GMD. | 2M | CO 2 | BL 1 |
|  | Define critical visual voltage. | 2M | CO 3 | BL 1 |
|  | What is Sag? List out various factors affecting the sag. | 2M | CO 2 | BL 1 |
|  | Compare DC and AC distribution systems with any one aspect. | 2M | CO 4 | BL 1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | a) Draw the schematic diagram of a Thermal power station and explain its operation. | 6M | CO-1 | BL-2 |
|  | b) Explain the essential factors which influence the choice of site for a hydel power plant. | 4M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2. | a) Draw the schematic diagram of a nuclear power station and discuss its operation. | 6M | CO-1 | BL-2 |
|  | b) List the advantages of gas power plant. | 4M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3. | a) Explain Skin and Proximity effects briefly. | 5M | CO-2 | BL-2 |
|  | b) Derive the expression for inductance of a 3-phase transmission line with symmetrically spaced conductors. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 4. | A 3-phase, 50 Hz transmission line 100 km long delivers 20 MW at 0·9 p.f. lagging and at 110 kV. The resistance and inductive reactance of the line per phase per km are 0·2 Ω and 0·4 Ω respectively, while capacitance admittance is 2·5 × 10− 6 siemen/km/phase. Calculate: (i) the current and voltage at the sending end (ii) efficiency of transmission line. Use nominal T method. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |  |
| 5. | a) Explain about Bewley’s Lattice diagram with suitable example. | 5M | CO-3 | BL-2 |
|  | b) Define Corona and also factors affecting corona. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 6. | a) Derive reflection and refraction coefficient of a transmission line when receiving end is open circuited. | 5M | CO-3 | BL-3 |
|  | b) A 3-phase transmission line has conductors 2 cm in diameter spaced equilaterally 1 m apart. If the dielectric strength of air (g0) is 21.2 kV (rms) per cm, find the disruptive critical voltage for the line. Take air density factor δ = 0·952 and irregularity factor mo = 0·9. | 5M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |  |
| 7. | a) A transmission line has a span of 150 m between level supports. The conductor has a cross-sectional area of 2 cm2. The tension in the conductor is 2000 kg. If the specific gravity of the conductor material is 9·9 gm/cm3 and wind pressure is 1·5 kg/m length, calculate the sag. What is the vertical sag? | 5M | CO-2 | BL-4 |
|  | b) Define String efficiency? Explain various methods of improving string efficiency in brief. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 8. | Each line of a 3-phase system is suspended by a string of 3 identical insulators of self-capacitance C farad. The shunt capacitance of connecting metal work of each insulator is 0·2 C to earth and 0·1 C to line. Calculate the string efficiency of the system if a guard ring increases the capacitance to the line of metal work of the lowest insulator to 0·3 C. | 10M | CO-2 | BL-4 |
| UNIT-V |  |  |  |  |  |
| 9. | A 2-wire d.c. ring distributor is 300 m long and is fed at 240 V at point A. At point B, 150 m from A, a load of 120 A is taken and at C, 100 m in the opposite direction, a load of 80 A is taken. If the resistance per 100 m of single conductor is 0·03 Ω, Find: (i) current in each section of distributor (ii) voltage at points B and C | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |  |
| 10. | a) What are the design considerations of feeders and distributors in a Distribution System? | 4M | CO-4 | BL-2 |
|  | b) What are the requirements of a distribution system? | 6M | CO-4 | BL-2 |

***