| Subject Code 22PC1CE204 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular Examinations, July 2024

STRENGTH OF MATERIALS-II

(CE)

Time: 3 hours                                                                                 		      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Define principal stresses and principal planes. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Classify the types of springs. | 2M | CO-2 | BL-2 |
|  | Define compound cylinder. | 2M | CO-3 | BL-1 |
|  | Define slenderness ratio. | 2M | CO-4 | BL-1 |
|  | Define core or kernel of a section. | 2M | CO-4 | BL-1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | A rectangular block of material is subjected to a tensile stress of 110 N/mm2 on one plane and a tensile stress of 47 N/mm2 on the plane right angles to the former. Each of the above stresses is accompanied by a shear stress of 63 N/mm2. To Find the following (i) direction and magnitude of Principal stress and (ii) magnitude of maximum shear stress. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | The principal stresses at a point across two perpendicular planes are 100 N/mm2 (tensile) and 45 N/mm2 (tensile). Find the normal, tangential stresses and the resultant stress and its obliquity on a plane at 200 with the major principal plane. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | Derive the torsion equation for the circular shaft. | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
| 4. | A closely coiled helical spring of round steel wire 10 mm in diameter having 10 complete turns with a mean diameter of 120 mm is subjected to an axial load of 200 N. Determine i) The deflection of the spring ii) Maximum shear stress in the wire iii) Stiffness of the spring. Take (C)=8x104 N/mm2. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | A cylindrical shell, 0.8 m in diameter and 3 m long is having 10 mm wall thickness. If the shell is subjected to an internal pressure of 2.5 N/mm2, Calculate (a) change in diameter; (b) change in length; (c) change in volume. Take E = 200 GPa and Poisson's ratio = 0.25. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | Find the thickness of metal necessary for a cylindrical shell of internal diameter 160 mm to withstand an internal pressure of 8 N/mm2. The maximum hoop stress in the section is not to exceed 35 N/mm2. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | - Write assumption of Euler’s column buckling. | 4M | CO-4 | BL-2 |
|  | - Formulate the equation for the Euler’s critical load of a column when both ends are hinged. | 6M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 8. | Compare the crippling loads given by Euler’s and Rankine’s formula for a tubular steel strut of outer diameter 40 mm and wall thickness 4 mm. Length of the strut is 2.5 m. fc=320 N/mm2, α = 1/7500, E=2.1 x 105 N/mm2. The strut is fixed at both ends. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | Determine the maximum and minimum stresses at the base of a hallow circular chimney of height 20m with external diameter 4m and internal diameter 2m. The chimney is subjected to a horizontal wind pressure of 1kN/m2. The specific weight of the material of chimney is 22 kN/m3. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10. | Determine the diameter of a bolt which is subjected to an axial pull of 12 KN together with a transverse Shear Force of 6 KN using i) Maximum principal stress theory ii) Maximum principal strain theory. Elastic limit in tension is 225 N/mm2, factor of safety=3 and Poisson’s ratio = 0.3. | 10M | CO-4 | BL-2 |

***