| Subject Code 22PC1EC202 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular Examinations, July 2024

SWITCHING THEORY AND LOGIC DESIGN

(EIE)

Time: 3 hours                                                                                 		               Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Distinguish Weighted and Non-Weighted codes. | 2M | CO-1 | BL-4 |
| --- | --- | --- | --- | --- |
|  | Explain minimal SOP and POS forms. | 2M | CO-2 | BL-2 |
|  | What are the advantages and disadvantages of using a PROM as PLD? | 2M | CO-3 | BL-2 |
|  | Give the characteristic and excitation tables of JK F/F. | 2M | CO-4 | BL-1 |
|  | List the applications of registers | 2M | CO-5 | BL-1 |

PART-B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) Discuss with a neat circuit diagram working of TTL NAND Gate. | 5M | CO-1 | BL-2 |
|  | b) Subtract the following numbers using 10’s complement (i) 5250 – 321 (ii) 753 – 864 | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | a) Convert the following numbers. i) (1984)10 to base 8 ii) (53.1575)10 to base 2 iii) (3A6)16 to decimal | 5M | CO-1 | BL-2 |
|  | b) Obtain the 1’s and 2’s complement of the following binary numbers i) 1010101 ii) 0111000 iii)0000001 iv)10000 v) 00000 | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | a) Simplify the following Boolean functions using Karnaugh maps to a minimum number of terms. i) F(A,B,C,D) = ∑m( 0,1,2, 4, 6, 10, 14) ii) F(a, b, c) = ∑m(1,2,5,7) + d (0). | 5M | CO-2 | BL-4 |
|  | b) Simplify the following expression F(w,x,y,z)=∑m(2,3,10,11,12,13,14,15) and implement using minimum number of 2 input NAND gates. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | - Use the Tabulation method to simplify the following function F(A,B,C,D)=∑m(1,4, 6,7,8,9,10,11,15). | 6M | CO-2 | BL-3 |
|  | - State and prove DeMorgan’s Theorem and Consensus Theorem | 4M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | a) Design a 2 bit magnitude comparator. | 5M | CO-3 | BL-4 |
|  | b) Implement the following function using PLA. A(x,y,z)=∑m(1,2,4,6) B(x,y,z)=∑m(0,1,6,7) C(x,y,z)= ∑m(2,6) | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | Implement the following Boolean function with 8:1 Multiplexer. F(A,B,C,D)=∑m(0,2,6,10,11,12,13). | 5M | CO-3 | BL-3 |
|  | b) Design a combinational circuit using a ROM. The circuit accepts 3-bit binary number and generates its equivalent Gray code. | 5M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | a) Explain the working of 4 bit Binary Asynchronous counter. | 5M | CO-4 | BL-2 |
|  | b) Obtain the following realizations. Use K maps for the design (i)JK flipflop from SR flipflop (ii)D flipflop from JK flipflop | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8. | a) Draw the circuit diagram of a JK master slave flipflop using NAND gates and explain its operation. Discuss how race around condition is eliminated in this flipflop. | 5M | CO-4 | BL-4 |
|  | b) Design a Synchronous Decade Counter. | 5M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | - Draw the diagram of Universal Shift Register and explain in detail. | 6M | CO-5 | BL-2 |
|  | - What is Moore and Mealy machine? Compare them. | 4M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10. | Reduce the number of states in the following state table and tabulate the reduced state table and give proper state assignment. PS; NS,Z. X=0; X=1. A; A,0; B,0. B; C,0; D,0. C; A,0; D,0. D; E,0; F,1. E; A,0; F,1. F; E,0; F,1. G; A,0; F,1. | 10M | CO-5 | BL-3 |

***