| Subject Code 22PC1AE207 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular Examinations, July 2024

THEORY OF MACHINES AND MECHANISMS

(AE)

Time: 3 hours                                                                                 		              Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A

5X2=10M

|  | Write the inversions of the four-bar mechanism. | 2M | CO- 1 | BL 1 |
| --- | --- | --- | --- | --- |
|  | Write the relation between linear velocity & angular velocity in mechanisms. | 2M | CO- 1 | BL 1 |
|  | Write the applications of cam- followers. | 2M | CO- 2 | BL 1 |
|  | Define the law of gearing. | 2M | CO- 3 | BL 1 |
|  | In a Hartnell governor, if a spring of greater stiffness is used, then the governor will be less sensitive, justify? | 2M | CO- 4 | BL 2 |

PART-B

5X10=50M

| UNIT-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | - Calculate the mobility of the following mechanisms. Fig. 1(i) Fig. 1(ii) | 6M | CO-1 | BL-2 |
|  | - Identify the nature of the mechanism shown in Fig. 2 by using Grashof’s law. Fig. 2 | 4M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
| 2. | a) Explain the functioning of a crank and slotted lever quick return motion mechanism. b) In a crank and slotted lever quick-return mechanism, the distance between the fixed centers is 300 mm and the length of the driving crank is 150 mm. Find the inclination of the slotted lever with the vertical in the extreme position and the ratio of time of cutting stroke to return stroke. | 4M 6M | CO-1 CO-1 | BL-2 BL-3 |
| UNIT-II |  |  |  |  |  |
| 3. | - Describe the Klien’s Construction. | 5M | CO-1 | BL-1 |
|  | - Write the Properties of the Instantaneous Centre and how to calculate the number of Instantaneous Centres in a mechanism. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 4. | In a four-bar chain ABCD, AD is fixed and is 120 mm long. The crank AB is 50 mm long and rotates at 150 r.p.m. clockwise, while the link CD = 100 mm oscillates about D. BC and AD are of equal length. Find the angular velocity of link CD when angle BAD = 45°. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5. | a) List out the types of Cam-follower mechanisms & write the applications of cam & follower. | 4M | CO-2 | BL-2 |
|  | b) A ship is propelled by a turbine rotor of mass 500 kg and has a speed of 2400 rpm. The rotor has a radius of gyration of 0.5 m and rotates in a clockwise direction when viewed from the stern. Find the gyroscopic effect on the ship if it runs at a speed of 15 knots (1 knot=1860 m/h). It steers to the left in a curve of 60 m radius. | 6M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 6. | Construct the cam-follower diagram of a knife edge with a maximum displacement of follower is 50 mm. It follows a velocity with a simple harmonic motion. The follower rises through 45o, then dwells for 30o, returns with 120o, and remains in the dwell position | 10M | CO- 2 | BL-2 |
| UNIT-IV |  |  |  |  |  |
| 7. | a) Describe the Interference in Involute Gears, and how it can be prevented. | 5M | CO-3 | BL-2 |
|  | b) Determine the minimum number of teeth required on a pinion, to avoid interference which is to gear with, i) Wheel to give a gear ratio of 3 to 1. ii) An equal wheel. The pressure angle is 20° and a standard addendum of 1 module for the wheel may be assumed (aw=1). | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 8. | In a reverted epicyclic gear train, arm A carries two gears B and C, and a compound gear D - E. Gear B meshes with gear E and gear C meshes with gear D. The number of teeth on gears B, C, and D are 75, 30 and 90 respectively, shown in Fig. 3. Find the speed and direction of gear C when gear B is fixed and the arm A makes 100 r.p.m. clockwise. Fig, 3 | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |  |
| 9. | a) Differentiate the Watt & Porter governors? | 5M | CO-4 | BL-4 |
|  | b) A Porter governor has equal arms each 250 mm long and pivoted on the axis of rotation. Each ball has a mass of 5 kg and the mass of the central load on the sleeve is 25 kg. The radius of rotation of the ball is 150 mm when the governor begins to lift and 200 mm when the governor is at maximum speed. Find the minimum and maximum speeds and range of speed of the governor. | 5M | CO-4 | BL-4 |
| OR |  |  |  |  |  |
| 10. | State the different types of governors. And explain the following terms relating to governors: 1. Stability, 2. Sensitiveness, 3. Isochronism, and 4. Hunting. | 10M | CO-4 | BL-2 |

***