| Subject Code A19PE1CE01 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, January 2024

ADVANCED STRUCTURAL ANALYSIS

(CE)

Time: 3 hours                                                                                 		      		Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Mention the types of arches based on structural behavior. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Define carry over factor. | 2M | CO-2 | BL-2 |
|  | Write any two advantages of kani’s method. | 2M | CO-2 | BL-2 |
|  | List any two properties of flexibility matrix. | 2M | CO-4 | BL-1 |
|  | What does the stiffness coefficient kij? | 2M | CO-4 | BL-2 |

PART-B

6X10=60M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | A two hinged parabolic arch of constant cross section has a span of 60m and a rise of 10m. It is subjected to loading as shown in Fig1. Calculate reactions of the arch if the temperature of the arch is raised by 40°C. Assume co-efficient of thermal expansion as α = 12×10−6 / °C. Fig 1 | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
|  | Determine the length of the cable and max tension developed if the cable supports a load of 2kN/m on a horizontal span of 300m as shown in Fig 2. The maximum sag is 25m. Fig.2 | 10M | CO-1 | BL-3 |
| UNIT-2 |  |  |  |  |
|  | Obtain the slope deflection equations from first principles. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | Analyze the continuous beam shown in Fig. 3 by slope deflection method and draw BMD. Show salient values on the BMD. Span A & D are fixed and B&C are simply supported. Span CD is 3m and is subjected to a 20kN concentrated load at mid span. EI is constant. Fig.3 | 10M | CO-2 | BL-3 |
| UNIT-3 |  |  |  |  |
|  | Analyze the continuous beam shown in Fig 4 by moment distribution method and draw BMD. The support 2 sinks by 10mm under the loading. Take E=200x106kN/m2 and I=80x10-6m4. Show salient values on the Bending moment diagram. Fig .4 | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | Analyse the portal frame shown in Fig.5 using Moment Distribution Method and draw BMD. Fig. 5 | 10M | CO-3 | BL-4 |
| UNIT-4 |  |  |  |  |
|  | Analyse the continuous beam shown in Fig 6 using kani’s method. Draw BMD. Fig.6 | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | Analyse the portal frame shown in Fig 7 using Kani’s Method. Supports A&D are fixed. Fig .7 | 10M | CO-3 | BL-4 |
| UNIT-5 |  |  |  |  |
|  | Develop the stiffness matrix for the beam shown in Fig 8 with respect to the coordinates shown in Figure. EI is constant. Fig .8 | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | Analyse the two span continuous beam shown in Fig. 9 using stiffness method. The moment of inertia of span AB is 2I and span BC is I. Fig.9 | 10M | CO-4 | BL-3 |
| UNIT-6 |  |  |  |  |
|  | Develop the flexibility matrix for the beam shown in Fig.10 with respect to the coordinates shown in Figure. EI is constant. Fig.10 | 10M | CO-4 | BL-3,4 |
| OR |  |  |  |  |
|  | Analyse the continuous beam shown in Fig.11 using flexibility method. EI is constant. Fig.11 | 10M | CO-4 | BL-4 |

***