| Subject Code A19PC1EC08 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, January 2024

ANTENNAS AND WAVE PROPAGATION

(ECE)

Time: 3 hours                                                                                 		     	     Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Define the terms beam width and polarization of an antenna. | 2M | CO1 | BL2 |
| --- | --- | --- | --- | --- |
|  | Compare loop antenna with dipole. | 2M | CO2 | BL2 |
|  | What is the advantage of Binomial arrays? | 2M | CO3 | BL1 |
|  | Define spill over and Back lobes. | 2M | CO2 | BL1 |
|  | What is radio Horizon? | 2M | CO4 | BL1 |

PART-B

6X10=60M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Define and explain the terms Radiation intensity, Beam Efficiency, Directivity Effective aperture and effective height of an antenna. | 10M | CO1 | BL2 |
| OR |  |  |  |  |
|  | Explain the different methods to measure gain of an antenna. | 10M | CO2 | BL2 |
| UNIT-2 |  |  |  |  |
|  | Prove that the Radiation Resistance of an Quarter Wave Monopole is 37.5Ω. | 10M | CO1 | BL5 |
| OR |  |  |  |  |
|  | Derive the expression for electromagnetic field radiated by an alternating current element and compute its radiation resistance. | 10M | CO1 | BL5 |
| UNIT-3 |  |  |  |  |
|  | a) Derive an expression for Beam Width using End-fire array. b) With a neat sketch explain the operation of Yagi-Uda array. | 5M 5M | CO3 CO3 | BL5 BL3 |
| OR |  |  |  |  |
|  | a) Distinguish resonant and non-resonant antennas. b) What is a Patch? Explain the operating principle of microstrip antenna and discuss its characteristics , applications and advantages. | 3M 7M | CO3 CO3 | BL2 BL2 |
| UNIT-4 |  |  |  |  |
|  | Explain the working principle of Helical antenna in Axial and normal modes. | 10M | CO2 | BL2 |
| OR |  |  |  |  |
|  | a) With a neat sketch, Explain the Cassegrain mechanism in transmission mode. b) For a 20 turn helical antenna operating at 3GHz with circumference of 10cm and spacing between the turns 0.3λ is operating at 3GHz. Calculate the directivity and half power beam width. | 5M 5M | CO2 CO2 | BL2 BL3 |
| UNIT-5 |  |  |  |  |
|  | a) Explain the effects of earth’s properties on ground wave propagation. b) For a flat earth assume that at 400km reflection takes place. The maximum density of the ionosphere corresponds to a refractive index of 0.9 at 10 MHz. Calculate skip distance of which fMUF=10MHz, height=90km, fc=5MHz. | 5M 5M | CO4 CO4 | BL3 BL3 |
| OR |  |  |  |  |
|  | Describe the characteristics of different layers existing in ionosphere. Explain the terms critical frequency, MUF, LUF, Optimum working frequency and skip distance. | 10M | CO4 | BL3 |
| UNIT-6 |  |  |  |  |
|  | Determine the expression for range of space wave propagation. | 10M | CO4 | BL3 |
| OR |  |  |  |  |
|  | a) Explain the effective earth’s radius in tropospheric wave propagation. b) Briefly explain about duct propagation. | 5M 5M | CO4 CO4 | BL2 BL2 |

***