| Subject Code A19PC1AE07 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, January 2024

DESIGN OF AUTOMOTIVE COMPONENTS-I

(AE)

Time: 3 hours                                                                                 		      	Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Define factor of safety. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is endurance limit? | 2M | CO-2 | BL-2 |
|  | Draw the symbol for double V butt joint and edge joint | 2M | CO-3 | BL-1 |
|  | Write the advantages of flexible drives over rigid drives. | 2M | CO-4 | BL-1 |
|  | What is mean by critical speed of shaft? | 2M | CO-4 | BL-1 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Explain the mechanical properties of engineering materials. | 10M | CO-1 | BL-1 |
| OR |  |  |  |  |  |
|  | Discuss the maximum principal stress theory and maximum shear stress theory. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
|  | Explain with neat sketches Soderberg and Goodman lines for fatigue strength. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
|  | Discuss the S-N diagrams for steels | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |  |
|  | Discuss about tap bolts, cap screws and studs with sketches. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
|  | A steel plate subjected to a force of 5 kN and fixed to a channel by means of three identical bolts is shown in Fig. 1. The bolts are made of plain carbon steel 30C8 (Syt = 400 N/mm2) and the factor of safety is 3. Determine the diameter of the shank. Fig.1 | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |  |
|  | A low carbon steel plate of 0.7 m width welded to a structure of similar material using two parallel fillet welds of 0.112 m length (each) is subjected to an eccentric load of 4000 N, the line of action of which has a distance of 1.5 m from the centre of gravity of the weld group. Design the required thickness of the plate when the allowable stress of the weld metal is 60 MPa and that of the plate is 40 MPa. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |  |
|  | A plate, 75 mm wide and 10 mm thick, is joined with another steel plate by means of single transverse and double parallel fillet welds, as shown in fig.2. The joint is subjected to a maximum tensile force of 55 kN. The permissible tensile and shear stresses in the weld material are 70 and 50 N/mm2 respectively. Determine the required length of each parallel fillet weld. Fig. 2 | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |  |
|  | It is required to design a helical compression spring subjected to a maximum force of 7.5 kN. The main coil diameter should be 150 mm from space consideration. The spring rate is 75 N/mm. The spring is made of oil-hardened and tempered steel wire with ultimate tensile strength of 1250 N/mm2.The permissible shear stress for the spring wire is 30% of the ultimate tensile strength (G=81370 N/mm2). Calculate wire diameter and number of active coils. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |  |
|  | The following data is given for an open-type V belt drive: diameter of driving pulley = 150 mm, diameter of driven pulley = 300 mm, centre distance = 1 m, groove angle = 40°, mass of belt = 0.25 kg/m, maximum permissible tension = 750 N, coefficient of friction = 0.2. Plot a graph of maximum tension and power transmitted against the belt velocity. Calculate the maximum power transmitted by the belt and the corresponding belt velocity. Neglect power losses. | 10M | CO-4 | BL-4 |
| UNIT-VI |  |  |  |  |  |
|  | A solid shaft diameter ‘d’ is used in power transmission. Due to modification of existing transmission system, it is required to replace to solid shaft by a hollow shaft of the same material and equally strong in torsion. Further, the weight of hollow shaft per meter length should be half of the solid shaft. Determine the outer diameter of hollow shaft in terms of ‘d’. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |  |
|  | A hollow transmission shaft, having inside diameter 0.6 times outside diameter, is made of plain carbon steel 40C8 (Syt = 380 N/mm2) and the factor of safety is 3. A belt pully, 1000 mm in diameter, is mounted on the shaft, which over hangs the left hand bearing by 250 mm. The belts are vertical and transmit power to the machine shaft below the pully. The tension on the tight and slack sides of the belt one 3kN and 1kN respectively, while the weight of the pully is 500 N. The angle of wrap of the belt on the pully is 1800. Calculate the outer and inner diameter of the shaft. | 10M | CO-4 | BL-3 |

***