| Subject Code A19PC1EC09 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, January 2024

DIGITAL SIGNAL PROCESSING

(ECE)

Time: 3 hours                                                                                 		      	Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | How are discrete -time systems analysed using Z-transforms? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Why DFT is preferred over DTFT? | 2M | CO-1 | BL-1 |
|  | What is the importance of poles in filter design? | 2M | CO-2 | BL-1 |
|  | Write any two characteristic features of rectangular window. | 2M | CO-2 | BL-1 |
|  | What is Sampling rate conversion? | 2M | CO-3 | BL-1 |

PART-B

6X10=60M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Determine the impulse response and step response of the causal system given below and discuss stability. yn-yn-1-2(yn-2=xn-1+2x(n-2) | 10M | CO-1 | BL-4 |
| OR |  |  |  |  |
|  | Discuss about the frequency domain representation of the signals and systems. | 10M | CO-1 | BL-4 |
| UNIT-2 |  |  |  |  |
|  | Compute the 8-point DFT of xn={1,1,0,0,0,0,0,0}, use DIFFFT algorithm. | 10M | CO-1 | BL-4 |
| OR |  |  |  |  |
|  | Find the 8-point DFT by radix-2 DIT FFT algorithm. xn={2,1,2,1,2,1,2,1} | 10M | CO-1 | BL-4 |
| UNIT-3 |  |  |  |  |
|  | Design a Butterworth Filter Using the Impulse Invariance method for the following. Specifications 0.8≤H(ejω)≤1; 0≤ω≤0.2π H(ejω)≤0.2; 0.6π≤ω≤π | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
|  | Obtain the Direct form-I, Direct form- II, Cascade and Parallel form realization for the system. yn=-0.1yn-1+0.2yn-2+3xn+3.6xn-1+0.6x(n-2) | 10M | CO-2 | BL-4 |
| UNIT-4 |  |  |  |  |
|  | Design a filter with Hejω=e-j3ω; -π4≤ω≤π4 = 0; π4≤ω≤π Using a Hamming window with N=7. | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
|  | a) Explain FIR filter design using Windowing Method b) Realize the FIR system using the direct form structure. yn=3xn+5xn-1-2x(n-2). | 5M 5M | CO-2 CO-2 | BL-2 BL-4 |
| UNIT-5 |  |  |  |  |
|  | Consider the signal xn=n u(n) - Plot the Original and Interpolated Signal. - Compute and draw the spectrum of original signal. - Compute and draw the spectrum of Interpolated signal. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
|  | a) Explain the Interpolation process and its frequency domain representation. b) Implement the Interpolation using polyphase structures. | 5M 5M | CO-3 CO-3 | BL-2 BL-2 |
| UNIT-6 |  |  |  |  |
|  | a) Discuss about the effects of truncation of IIR filter coefficients on filter response. b) Explain about the formation of Dead band in IIR Filters. | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
|  | a) Explain about the Computational output round-off errors in digital filters. b) Discuss about the overflow limit cycles and the methods to prevent overflow. | 5M 5M | CO-4 CO-4 | BL-2 BL-2 |

***