| Subject Code A19PC1ME11 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, January 2024

DYNAMICS OF MACHINERY

(ME)

Time: 3 hours                                                                                 		      	     Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | What do mean by applied and constrained forces? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Briefly explain gyroscopic couple. | 2M | CO-3 | BL-1 |
|  | Define the terms coefficient of fluctuation of energy and coefficient of fluctuation of speed. | 2M | CO-3 | BL-1 |
|  | What is the advantage of self-expanding shoe brake? | 2M | CO-2 | BL-1 |
|  | Distinguish between transverse and torsional vibrations. | 2M | CO-4 | BL-1 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | A slider crank mechanism with the following dimensions is acted upon by a force F=2kN at B as shown in figure. OA=100mm, AB=450mm. Determine the input torque T on the link OA for the static equilibrium of the mechanism for the given configuration | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
|  | The dimensions of a four-link mechanism are AB = 500mm, BC = 660mm, CD = 560 mm, AD = 1000mm. The link AB has an angular velocity of 10.5 rad/s counter-clockwise and an angular retardation of 26 rad/s2 at the instant when it makes an angle of 600 with AD, the fixed link. The mass of the links BC and CD is 4.2 kg/m length. The link AB has a mass of 3.54 kg, the centre of which lies at 200 mm from A and a moment of inertia of 88500 kg.mm2. Neglecting gravity and friction effects, determine the instantaneous value of the drive torque required to be applied on AB to overcome the inertia forces. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
|  | a) Define Gyroscopic couple and explain the effect on aeroplane, when the aeroplane is taking left and right turns, engine is rotating in anticlockwise direction and view point of the observer is from front. | 5M | CO-3 | BL-2 |
|  | b) The heavy turbine rotor of a sea vessel rotates at 1500 rpm. clockwise looking from the stern, its mass being 750 kg. The vessel pitches with an angular velocity of 1 rad/s. Determine the gyroscopic couple transmitted to the hull when bow is rising, if the radius of gyration for the rotor is 250 mm. Also show in what direction the couple acts on the hull? | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | a) What do you understand by gyroscopic couple? Derive a formula for its magnitude. | 5M | CO-3 | BL-2 |
|  | b) An Aeroplane makes a complete half circle of 50m radius, towards left, when flying at 200 km/hr. The rotary engine and the propeller of the plane have a mass of 400kg and a radius of gyration of 0.3m. The engine rotates at 2400rpm. clockwise when viewed from the rear. Find the gyroscopic couple on the aircraft and state its effect on it. | 5M | CO-3 | BL-3 |
| UNIT-III |  |  |  |  |
|  | The turning moment diagram for a multicylinder engine has been drawn to a scale 1mm = 600 N-m vertically and 1 mm = 3° horizontally. The intercepted areas between the output torque curve and the mean resistance line, taken in order from one end, are as follows: + 52, – 124, + 92, – 140, + 85, – 72 and + 107 mm2, when the engine is running at a speed of 600 rpm. If the total fluctuation of speed is not to exceed ± 1.5% of the mean speed, find the necessary mass of the flywheel of radius 0.5 m. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | In a Hartnell governor, the lengths of ball and sleeve arms of a bell crank lever are 120 mm and 100 mm respectively. The distance of the fulcrum of the bell crank lever from the governor axis is 140mm. Each governor ball has a mass of 4 kg. The governor runs at a mean speed of 300rpm. with the ball arms vertical and sleeve arms horizontal. For an increase of speed of 4 per cent, the sleeve moves 10 mm upwards. Neglecting friction, find: (i) the minimum equilibrium speed if the total sleeve movement is limited to 20 mm, (ii) the spring stiffness, (iii) the sensitiveness of the governor, and (iv) the spring stiffness if the governor is to be isochronous at 300 rpm. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
|  | A centrifugal clutch is to transmit 15 kW at 900 rpm. The shoes are four in number. The speed at which the engagement begins is 3/4th of the running speed. The inside radius of the pulley rim is 150 mm and the centre of gravity of the shoe lies at 120 mm from the centre of the spider. The shoes are lined with Ferrodo for which the coefficient of friction may be taken as 0.25. Determine: (i) Mass of the shoes, and (ii) Size of the shoes, if angle subtended by the shoes at the centre of the spider is 60º and the pressure exerted on the shoes is 0.1 N/mm2 | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
|  | a) Discuss various types of brakes and what is the self-locking brake. | 4M | CO-2 | BL-2 |
|  | b) A band brake acts on the 3/4th of circumference of a drum of 450mm diameter which is keyed to the shaft. The band brake provides a braking torque of 225 N-m. One end of the band is attached to a fulcrum pin of the lever and the other end to a pin 100 mm from the fulcrum. If the operating force is applied at 500 mm from the fulcrum and the coefficient of friction is 0.25, find the operating force when the drum rotates in the (i) anticlockwise direction, and (ii) clockwise direction. | 6M | CO-2 | BL-4 |
| UNIT-V |  |  |  |  |
|  | a) Explain clearly the terms ‘static balancing’ and ‘dynamic balancing’. State the necessary conditions to achieve them. | 4M | CO-2 | BL-2 |
|  | b) Four masses m1, m2, m3 and m4 are 200 kg, 300 kg, 240kg and 260kg respectively. The corresponding radii of rotation are 0.2m, 0.15m, 0.25m and 0.3m respectively and the angles between successive masses are 45°, 75° and 135°. Find the position and magnitude of the balance mass required, if its radius of rotation is 0.2 m. | 6M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | An inside cylinder locomotive has its cylinder centre lines 0.7 m apart and has a stroke of 0.6 m. The rotating masses per cylinder are equivalent to 150 kg at the crank pin, and the reciprocating masses per cylinder to 180 kg. The wheel center lines are 1.5 m apart. The cranks are at right angles. The whole of the rotating and 2/3 of the reciprocating masses are to be balanced by masses placed at a radius of 0.6 m. Find the magnitude and direction of the balancing masses. Find the fluctuation in rail pressure under one wheel, variation of tractive effort and the magnitude of swaying couple at a crank speed of 300 rpm. | 10M | CO-2 | BL-4 |
| UNIT-VI |  |  |  |  |
|  | a) Explain the term 'Logarithmic decrement' as applied to damped vibrations. | 4M | CO-4 | BL-2 |
|  | b) The following data are given for a vibratory system with viscous damping: Mass = 2.5 kg; spring constant = 3 N/mm and the amplitude decreases to 0.25 of the initial value after five consecutive cycles. Determine the damping coefficient of the damper in the system. | 6M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | a) Describe the method of finding the natural frequency of torsional vibration for three rotor system. | 4M | CO-4 | BL-2 |
|  | b) A shaft of length 1.5 m is 0.095m in diameter for the first 0.6 m of its length, 0.06 m in diameter for the next 0.5 m of the length and 0.05 m in diameter for the remaining 0.4 m of its length. The shaft carries two rotors at two ends. The mass moment of inertia of the first rotor is 650 kg m2 whereas of the second is 212 kgm2. Determine the natural frequency of free torsional vibration of the system. The modulus of rigidity of shaft material may be taken as 80 GN/m2. | 6M | CO-4 | BL-3 |

***