| Subject Code A19PC1CE10 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, January 2024

ENGINEERING HYDROLOGY

(CE)

Time: 3 hours                                                                                 		      		Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Draw a bar chart showing percentages of the entire world water quantity. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Explain briefly about IDF curves. | 2M | CO-1 | BL-2 |
|  | Write the equations to measure evapotranspiration. | 2M | CO-2 | BL-1 |
|  | List the assumptions involved in the unit hydrograph theory. | 2M | CO-3 | BL-1 |
|  | List the various aquifer properties. | 2M | CO-4 | BL-1 |

PART-B

6X10=60M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | a) Explain the hydrologic cycle with a neat sketch. b) Explain any three engineering applications of hydrology. | 5M 5M | CO-1 CO-1 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | a) Explain briefly about the history of hydrology. b) Explain briefly about the agencies which collect the hydrological data in India. | 5M 5M | CO-1 CO-1 | BL-2 BL-2 |
| UNIT-2 |  |  |  |  |
|  | a) State the various types of rain gauges? Explain with a neat sketch any two type of recording rain gauges? b) A catchment may be idealized as circle of 50 km radius, with the centre at (0,0). Five rain gauges are located within his area at P (0,0), Q (40,0), R (0,-40), S (-40,0) and T (0,40). All distances are in km. During a month, the precipitation over the catchment area recorded by the rain gauges are 55, 76, 68, 40 and 63 mm respectively. Calculate the mean precipitation by the Thiessen polygon method. | 5M 5M | CO-1 CO-1 | BL-2 BL-3 |
| OR |  |  |  |  |
|  | a) Discuss the various forms of precipitation. b) Precipitation station ‘X’ has inoperative for part of a month during which a storm occurred. The respective storm totals at three surrounding stations A, B and C are 60, 48 and 37.5 mm. The normal annual precipitation amounts of station X, A, B and C are 650, 720, 580 and 380 mm respectively. Estimate the storm precipitation for station X. | 5M 5M | CO-1 CO-1 | BL-2 BL-3 |
| UNIT-3 |  |  |  |  |
|  | a) What is pan coefficient? Describe any two evaporation pans for measuring evaporation. b) For a storm of 3 hrs duration, the infiltration rates are given below. If the surface run-off is 3.2cm, determine the w-index and ɸ-index. Time (minutes); 30; 30; 30; 30; 30; 30. Rainfall Rate (cm/hr); 1.2; 3.1; 4.5; 3.0; 2.0; 1.1. | 5M 5M | CO-2 CO-2 | BL-2 BL-4 |
| OR |  |  |  |  |
|  | a) Explain the various methods to reduce the evaporation from the reservoirs. b) Distinguish between the following: i. PET and AET; ii. Interception and Depression storage; iii. ɸ-Index and w-Index. | 5M 5M | CO-2 CO-2 | BL-2 BL-3 |
| UNIT-4 |  |  |  |  |
|  | a) What is a hydrograph? Draw a single peaked hydrograph and explain its components. b) Describe the SCS-CN method of estimating peak runoff from a catchment. | 5M 5M | CO-3 CO-3 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | a) Explain the factors that affect the runoff from a catchment. b) Table below gives ordinates of 6-hr unit hydrograph. Derive ordinates of 3-hr unit hydrograph for the same catchment using S-curve method. Time (hr); 0; 3; 6; 9; 12; 15; 18; 21; 24. 6-hr UH (cumec); 0; 10; 20; 30; 40; 30; 20; 10; 0. | 5M 5M | CO-3 CO-3 | BL-2 BL-4 |
| UNIT-5 |  |  |  |  |
|  | a) Explain the rational method of computing the peak discharge of a small catchment? b) Distinguish between hydraulic and hydrologic method of flood routing. | 5M 5M | CO-3 CO-3 | BL-2 BL-3 |
| OR |  |  |  |  |
|  | a) Explain briefly about the reservoir routing method. b) Flood-frequency computations for a river, by using Gumbel’s method, yielded the following results. Compute the peak flood at 500 yrs return period. Return Period (T yrs); Peak Flood(m3/s). 50; 40809. 100; 46300. | 5M 5M | CO-3 CO-3 | BL-2 BL-4 |
| UNIT-6 |  |  |  |  |
|  | a) Distinguish between i. Aquiclude and Aquifuge ; ii. Darcy velocity and Actual velocity; iii. Aquifer and Perched aquifer. b) An unconfined aquifer has a thickness of 30 m, A fully penetrating 20 cm diameter well in this aquifer is pumped at a rate of 35 lit/s. The drawdown measured in two observation wells located at distances of 10m and 100 m from the well are 7.5 m and 0.5 m respectively. Determine the average hydraulic conductivity of the aquifer. At what distance from the well the drawdown is insignificant? | 5M 5M | CO-4 CO-4 | BL-2 BL-3 |
| OR |  |  |  |  |
|  | a) Derive expression for computing discharge of a well in unconfined aquifer fully penetrating as per Dupuit’s theory. b) A 400 mm diameter well fully penetrates a confined aquifer of permeability 40 m/day. The length of the strainer is 20 m. Under steady state of pumping the discharge from the well was 1500 lpm. Compute the draw down at the well if the radius of influence of the well is 400 m. | 5M 5M | CO-4 CO-4 | BL-2 BL-3 |

***