| Subject Code A19PC1ME13 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, January 2024

HEAT TRANSFER

(AE)

(Heat transfer data book is allowed)

Time: 3 hours                                                                                 		     	      Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Define: Fourier law of heat conduction. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Describe Biot number and their significance. | 2M | CO-4 | BL-2 |
|  | Define: Film Boling and Pool Boiling. | 2M | CO-2 | BL-3 |
|  | Brief about fouling and scaling of heat exchangers. | 2M | CO-2 | BL-2 |
|  | Describe: Irradiation and absorptivity. | 2M | CO-4 | BL-2 |

PART-B

6X10=60M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Derive the three-dimensional steady state heat conduction equation for Spherical coordinates. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
|  | - What are the modes of heat transfer and explain in detail. - A stainless-steel plate 2cm thick is maintained at a temperature of 550oC on one face and 50oC on the other. The thermal conductivity of stainless steel at 300oC is 19.1 W/mk. Compute the heat transferred through the material per unit area. | 6M 4M | CO-1 CO-1 | BL-2 BL-3 |
| UNIT-2 |  |  |  |  |
|  | A 4-m-high and 6-m-wide wall consists of a long 18-cm x 30-cm cross section of horizontal bricks (k-0.72 W/m·°C) separated by 3-cm-thick plaster layers (k-0.22 W/m·°C). There are also 2-cm-thick plaster layers on each side of the wall, and a 2-cm-thick rigid foam (k -0.026 W/m·°C) on the inner side of the wall. The indoor and the outdoor temperatures are 22°C and 4°C, and the convection heat transfer coefficients on the inner and the outer sides are h1 - 10 W/m2·°C and h2 - 20 W/m2·°C, respectively. Assuming one-dimensional heat transfer and disregarding radiation, determine the rate of heat transfer through the wall. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | In a food processing facility, a spherical container of inner radius r1- 40 cm, outer radius r2 - 41 cm, and thermal conductivity k -1.5 W/m·°C is used to store hot water and to keep it at 100°C at all times. To accomplish this, the outer surface of the container is wrapped with a 500-W electric strip heater and then insulated. The temperature of the inner surface of the container is observed to be nearly 100°C at all times. Assuming 10 percent of the heat generated in the heater is lost through the insulation, (a) express the differential equation and the boundary conditions for steady one-dimensional heat conduction through the container, and (b) evaluate the outer surface temperature of the container | 10M | CO-4 | BL-2 |
| UNIT-3 |  |  |  |  |
|  | Water is to be heated from 15°C to 65°C as it flows through a 3-cm-internal diameter5 m long tube. The tube is equipped with an electric resistance heater that provides uniform heating throughout the surface of the tube. The outer surface of the heater is well insulated, so that in steady operation all the heat generated in the heater is transferred to the water in the tube. If the system is to provide hot water at a rate of 10 L/min, determine the power rating of the resistance heater. Also, estimate the inner surface temperature of the pipe at the exit | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | A 25-cm-diameter stainless steel ball (ρ - 8055 kg/m3, Cp-480 J/kg·°C) is removed from the oven at a uniform temperature of 300°C. The ball is then subjected to the flow of air at 1 atm pressure and 25°C with a velocity of 3 m/s. The surface temperature of the ball eventually drops to 200°C. Determine the average convection heat transfer coefficient during this cooling process and estimate how long the process will take. | 10M | CO-3 | BL-3 |
| UNIT-4 |  |  |  |  |
|  | Water is boiled at atmospheric pressure by a horizontal polished copper heating element of diameter D- 5 mm and emissivity (ε) 0.05 immersed in water, as shown in Figure 2. If the surface temperature of the heating wire is 350°C, determine the rate of heat transfer from the wire to the water per unit length of the wire. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
|  | Discuss briefly the various regimes in boiling heat transfer with neat sketches. | 10M | CO-3 | BL-4 |
| UNIT-5 |  |  |  |  |
|  | Consider a cylindrical furnace with r0= H= 1 m. The top (surface 1) and the base (surface 2) of the furnace has emissivities ε1= 0.8 and ε2= 0.4, respectively, and are maintained at uniform temperatures T1 = 700 K and T2 = 500 K. The side surface closely approximates a blackbody and is maintained at a temperature of T3 = 400 K. Determine the net rate of radiation heat transfer at each surface during steady operation and explain how these surfaces can be maintained at specified temperatures. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | A thin aluminum sheet with an emissivity of 0.12 m both sides is placed between two very large parallel plates maintained at uniform temperatures of T1=750 K and T2=400 K. The emissivities of the plates are ξ1=0.8 and ξ2=0.7. Determine the net rate of radiation heat transfer between the two plates per unit surface area of the plates, and the temperature of the radiation shield in steady operation. | 10M | CO-4 | BL-2 |
| UNIT-6 |  |  |  |  |
|  | - Discuss the advantages of NTU method over the LMTD method of heat exchanger design. - What is the expression for LMTD in a parallel flow exchanger where the capacity rates for the hot and cold fluids are the same. | 3M 7M | CO-4 CO-4 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | A 2-shell passes and 4-tube passes heat exchanger is used to heat glycerin from 20°C to 50°C by hot water, which enters the thin-walled 2-cm-diameter tubes at 80°C and leaves at 40°C. The total length of the tubes in the heat exchanger is 60 m. The convection heat transfer coefficient is 25 W/m2·°C on the glycerin (shell) side and 160 W/m2·°C on the water (tube) side. Determine the rate of heat transfer in the heat exchanger (a) before any fouling occurs and (b) after fouling with a fouling factor of 0.0006 m2·°C/ W occurs on the outer surfaces of the tubes. | 10M | CO-4 | BL-2 |

***