| Subject Code A19PC1EI06 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, January 2024

INDUSTRIAL PROCESS CONTROL INSTRUMENTATION

(EIE)

Time: 3 hours                                                                                 		     	    Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Why is mathematical modelling of a system needed? | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What is viscosity. How is it measured. | 2M | CO-1 | BL-1 |
|  | State the importance of control valves. | 2M | CO-3 | BL-2 |
|  | List the advantages of pneumatic actuators. | 2M | CO-2 | BL-2 |
|  | Briefly explain with neat sketch feed forward control system. | 2M | CO-4 | BL-2 |

PART-B

6X10=60M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Discuss the pressure measurement using McLeod gauge and Ionization Gauge. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
|  | a) Explain the operation of I/P convertor for pressure measurement. b) Explain Bourdon tube pressure measurement system with neat sketch. | 5M 5M | CO-1 CO-1 | BL-2 BL-2 |
| UNIT-2 |  |  |  |  |
|  | Explain the working of any two non-contact type level control systems. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
|  | a) Explain the working of Hot wire anemometer with neat sketch. b) Demonstrate mathematical modelling of flow system. | 5M 5M | CO-1 CO-2 | BL-2 BL-3 |
| UNIT-3 |  |  |  |  |
|  | What is thermal lag? Demonstrate the mathematical modeling of the thermal system. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | a) Explain the working principle of stroboscope tachometer with neat sketch. b) What is relative humidity. How is it measured for ambient conditions. | 5M 5M | CO-1 CO-1 | BL-2 BL-2 |
| UNIT-4 |  |  |  |  |
|  | Discuss in detail PD, PI and PID control systems. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
|  | a) Explain Ziegler’s Nichols Tuning with process reaction curve method. b) What is Quarter amplitude method of tuning, Explain with Damped oscillation method. | 5M 5M | CO-2 CO-2 | BL-2 BL-2 |
| UNIT-5 |  |  |  |  |
|  | Explain the working of globe valves and butterfly valves. Discuss their advantages and characteristics. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
|  | a) Differentiate between hydraulic and pneumatic actuators. b) What is a directional control valve? How is it used to control piston in pneumatic cylinder? | 5M 5M | CO-3 CO-3 | BL-4 BL-3 |
| UNIT-6 |  |  |  |  |
|  | Why are multiple loops needed in a control system. Explain any two types of multi loop control systems. | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
|  | a) What is a model predictive control system? Explain its importance. b) Demonstrate split range control by considering a suitable example. | 5M 5M | CO-4 CO-4 | BL-2 BL-3 |

***