| Subject Code A19PC1EC10 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, January 2024

MICRO PROCESSORS AND MICRO CONTROLLERS

(Common to ECE and EIE)

Time: 3 hours                                                                                 		      	    Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | What is an Accumulator? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What are Assembler Directives? | 2M | CO-2 | BL-1 |
|  | List the three profiles of ARM v7 Architecture. | 2M | CO-1 | BL-1 |
|  | Compare Procedure and Macro. | 2M | CO-2 | BL-2 |
|  | Differentiate between minimum and maximum mode systems. | 2M | CO-2 | BL-1 |

PART-B

6X10=60M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Express the categories under which the instructions in the instruction set of the 8086 microprocessor are grouped. Explain the operation of any two instructions in each group. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
|  | a) Differentiate between minimum and maximum mode of operation of 8086 microprocessor, and sketch the maximum mode configuration. b) Draw the pin diagram of 8086 processor and explain all the signals. | 5M 5M | CO-2 CO-2 | BL-2 BL-2 |
| UNIT-2 |  |  |  |  |
|  | Draw the timing diagram for the memory read machine cycle of 8086. Explain the function of relevant signals and discuss how each signal changes in the process of the machine cycle. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | a) Write a program for 8086 microprocessor that multiplies two bytes and stores the result in memory. b) Explain the functioning of 8255 programmable peripheral interface and its modes. | 5M 5M | CO-2 CO-3 | BL-2 BL-3 |
| UNIT-3 |  |  |  |  |
|  | Distinguish between Cortex-M3 Processor and Cortex-M3-Based MCU. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | a) Draw the ARM Cortex-M3 Block diagram. b) Explain the mechanism of stack implementation in ARM Cortex-M3 processor. | 5M 5M | CO-4 CO-4 | BL-2 BL-2 |
| UNIT-4 |  |  |  |  |
|  | Construct and explain in detail the predefined memory map of Cortex M3. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | a) Explain the following instructions with examples (i) RSB (ii) BIC (iii) ASR of Cortex M3. b) Explain three special functions of ARM Cortex M3 processor. | 5M 5M | CO-4 CO-1 | BL-2 BL-2 |
| UNIT-5 |  |  |  |  |
|  | Identify the memory access attributes of Cortex-M3 Processor and explain the issues of pipelining in Cortex-M3 Processor. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
|  | a) State the various applications of ARM Cortex M3 processor. b) Explain the following ARM instructions with an example i) Move ii) Comparison iii) Multiply. | 4M 6M | CO-1 CO-2 | BL-2 BL-2 |
| UNIT-6 |  |  |  |  |
|  | The register contents of 8086 is given below. CS=5000H, DS=6000H, SS=A000H, ES=B000H, SI=2000H, DI=3000H, BP=1002H, SP=0002H, AX=0000H, BX=5200H, CX=2000H, DX=2000H. Calculate the effective address and physical address of the following instructions. (a) IMUL AX, [BP+BX-8D] (b) SBB AL, ES:[SI+5D] (c) PUSH AX (d) AND AH, [SI+42D] (e) CMPSB | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | a) Explain about exceptions, interrupts and the vector table in an ARM Cortex M3 processor. b) Outline the typical development flow of Cortex-M3 Processor. | 5M 5M | CO-3 CO-2 | BL-2 BL-2 |

***