| Subject Code A19PE1EC02 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, January 2024

MOS CIRCUITS

(ECE)

Time: 3 hours                                                                                 			Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Explain the short-channel effects? | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What is the advantage of using source degeneration resistor in Common source amplifier. | 2M | CO-2 | BL-2 |
|  | What is Strong inversion and weak inversion in CMOS circuits | 2M | CO-2 | BL-2 |
|  | Explain Noise margin. | 2M | CO-3 | BL-1 |
|  | Name the types of ROM. | 2M | CO-4 | BL-1 |

PART-B

6X10=60M

| UNIT-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Draw and explain about the capacitive model of MOS Transistor? | 10M | CO-1 | BL-4 |
| OR |  |  |  |  |  |
|  | a) Explain how MOSFET acts as a switch. b) Explain about Scaling of MOSFET | 5M 5M | CO-1 CO-1 | BL-4 BL-2 |
| UNIT-2 |  |  |  |  |  |
|  | Calculate the gain of a common source MOSFET Amplifier and draw the input – output characteristics with the small signal model for the saturation region. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
|  | a) Calculate the gain for the source follower MOSFET with its input-output characteristics. b) Suppose that in the source follower circuit given below, (W/L)1 = 20/0.5, I1 = 200 μA, VTH0 = 0.6 V, ǾF = 0.7 V, VDD = 1.2 V, μnCox = 50 μA/V2, and γ = 0.4 V1/2. (a) Calculate Vout for Vin = 1.2 V. | 5M 5M | CO-2 CO-2 | BL-3 BL-5 |
| UNIT-3 |  |  |  |  |  |
|  | What is the basic differential pair and explain the differential amplifier with different MOS loads? | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |  |
|  | a) Explain the common mode response of the differential amplifier. b) Explain about cascade current mirrors. | 5M 5M | CO-2 CO-2 | BL-2 BL-2 |
| UNIT-4 |  |  |  |  |  |
|  | Explain the two-stage op-amp with single ended output stage in detail. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
|  | a) Design the switched capacitor circuit. b) The circuit given below is designed for a nominal gain of 10, i.e., 1+R1/R2=10. Determine the minimum value of A1 for a gain error of 1%. | 5M 5M | CO-2 CO-2 | BL-3 BL-5 |
| UNIT-5 |  |  |  |  |  |
|  | Explain the Static and Dynamic behaviour of CMOS Inverter? | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
|  | a) Explain the switching characteristics of CMOS Inverter? b) Explain the logical effort of paths in MOS inverters. | 5M 5M | CO-3 CO-3 | BL-2 BL-2 |
| UNIT-6 |  |  |  |  |  |
|  | Discuss the operation of DRAM and explain the periodic refreshment mechanism in DRAM Circuits. | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
|  | a) Explain the operation of SRAM Cell and leakage current components? b) Explain the concept of Flash Memory. | 5M 5M | CO-4 CO-4 | BL-2 BL-2 |

***