| Subject Code A19PC1EE06 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, January 2024

POWER SYSTEMS-II

(EEE)

Time: 3 hours                                                                                 		      	    Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | How do we relate the elements & buses in a bus incidence matrix? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | When the generator buses are treated as load bus in gauss seidel method. | 2M | CO-2 | BL-3 |
|  | Give the classification of power system faults. | 2M | CO-2 | BL-2 |
|  | How the steady state stability can be improved? | 2M | CO-3 | BL-1 |
|  | What is meant by steady state stability limit? | 2M | CO-4 | BL-1 |

PART-B

6X10=60M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Built the bus impedance matrix for the network shown. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
|  | Find the YBUS by direct inspection method for the network shown in figure. The shunt and line admittance are given per unit. | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
|  | A 3-Bus power system with generation at Bus-1(slack bus), V1= 1.05∟00, Y12 = 10-j20, Y13 = 10-j30 p.u., Y23 = 16-j32 p.u., Y22 = Y12 + Y23, Y33 = Y13 +Y23 with P2 = -1.566 p.u., Q2 = -1.162 p.u., P3 = -1.4 p.u. and Q3 = -0.5 p.u. Using GS-method, determine the voltages at load buses 2 and 3 after one iteration. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
|  | a) Give classification of buses in load flow studies. b) Write the algorithm for NR method in rectangular form without PV bus. | 5M 5M | CO-2 CO-2 | BL-1 BL-1 |
| UNIT-III |  |  |  |  |
|  | 100MVA,11KV generator with X’’=0.20 p.u is connected through a transformer and line to a bus bar that supplies three identical motor as shown in fig. and each motor has X’’=0.20 p.u and X’=0.25 p.u on a base of 20MVA,33KV.the bus voltage at the motors is 33KV when a three phase balanced fault occurs at the point F. Calculate sub transient fault current in circuit breaker B. | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
|  | - How do you get the short circuit MVA from per unit impedance? - Prove that the transformer per unit reactance is same on primary and secondary sides. | 2M 8M | CO-2 CO-2 | BL-2 BL-2 |
| UNIT-IV |  |  |  |  |
|  | Derive the necessary equations to determine the fault current for single line ground fault. | 10M | CO-3 | BL-1 |
| OR |  |  |  |  |
|  | Show that positive and negative sequence currents equal in magnitude but out of phase by 180 deg in a L-L Fault. | 10M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
|  | A two-pole, 50 Hz, 11 kv turbo alternator has a rating of 100MW, power factor 0.85 lagging. The rotor has a moment of inertia of 10,000 kg-m2. Calculate a) Kinetic energy at rotor. b) H and M. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
|  | a) Derive the rotor dynamics of synchronous machine. b) Explain the various methods of improving steady state stability limit in a power system. | 5M 5M | CO-3 CO-3 | BL-2 BL-1 |
| UNIT-VI |  |  |  |  |
|  | Discuss the application of equal area criterion for transient stability analysis when the mechanical input is suddenly increased. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
|  | Discuss the solution of swing equation using point-by-point method. | 10M | CO-4 | BL-2 |

***