| Subject Code A19PE1EC20 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, January 2024

SENSORS AND ACTUATORS

(ECE)

Time: 3 hours                                                                                 		      		Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | What do you understand by the term transducer? | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Explain the term heat flux. | 2M | CO-2 | BL-3 |
|  | What is hall effect sensor? | 2M | CO-2 | BL-1 |
|  | What are the applications of fibre optics sensors? | 2M | CO-1 | BL-1 |
|  | Differentiate between hydraulic and pneumatic actuators. | 2M | CO-3 | BL-2 |

PART-B

6X10=60M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | a) What are strain gauges? How stress is calculated by measurement of strain. Explain? b) What is Piezoelectric effect? Explain the working of Quartz resonators. | 5M 5M | CO-2 CO-4 | BL-3 BL-2 |
| OR |  |  |  |  |
|  | a) What is the Inductive effect of current. Explain by taking example of an inductive sensor. b) Briefly explain about the working principle of Ultrasonic Sensors? | 5M 5M | CO-2 CO-2 | BL-2 BL-2 |
| UNIT-2 |  |  |  |  |
|  | a) Explain the working of Helium Low Temperature Thermometer. b) Explain the working principle behind dielectric Constant and Refractive Index Thermo-sensors. | 4M 6M | CO-4 CO-4 | BL-2 BL-2 |
| OR |  |  |  |  |
|  | a) Explain the working principle of Quartz Crystal Thermo-electric Sensors. b) What is acoustics? Explain with the working of Acoustic Temperature Sensor. | 5M 5M | CO-4 CO-1 | BL-2 BL-3 |
| UNIT-3 |  |  |  |  |
|  | a) Define eddy currents in context of sensors. What are various areas of applications of eddy current sensors. b) Demonstrate the working principle of electromagnetic flow meter. | 5M 5M | CO-4 CO-4 | BL-3 BL-2 |
| OR |  |  |  |  |
|  | What are SQUID sensors? Explain various areas of applications for SQUID sensors. | 10M | CO-4 | BL-2 |
| UNIT-4 |  |  |  |  |
|  | What is understand by the term total internal reflection? Explain the working of fibre optics sensor in context of total internal reflection. | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
|  | a) Explain the construction of an X ray detection sensor. b) Elaborate about the working principle of electrochemical cell. | 5M 5M | CO-2 CO-2 | BL-2 BL-3 |
| UNIT-5 |  |  |  |  |
|  | Illustrate various types of filters that are used to enhance the signal output from a sensor. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |
|  | Summarize working principle of any two automotive sensors with neat sketch. | 10M | CO-4 | BL-3 |
| UNIT-6 |  |  |  |  |
|  | a) What is stepping in context of stepper motors? How is it achieved? b) Explain various methods of actuation for direction control valves. | 5M 5M | CO-3 CO-3 | BL-3 BL-2 |
| OR |  |  |  |  |
|  | What are various types of bearings used in mechanical systems. Explain with neat sketches. | 10M | CO-3 | BL-2 |

***