| Subject Code A19PC1ME12 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | A19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular Examinations, January 2024

TURBO MACHINERY

(ME)

Time: 3 hours                                                                                 		      	     Max. Marks: 70

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

All Questions Carry Equal Marks.

PART-A

5X2=10M

|  | Define boiler draught. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Write the function and application of the condenser. | 2M | CO-2 | BL-1 |
|  | What is a steam turbine? Also define impulse and reaction turbines. | 2M | CO-3 | BL-1 |
|  | Write the different types of compressors. | 2M | CO-3 | BL-1 |
|  | What do you mean by the term gas turbine and how are gas turbines classified? | 2M | CO-4 | BL-1 |

PART-B

6X10=60M

| UNIT-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Explain the construction and working of Babcock and Wilcox boiler with a neat sketch. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
|  | Derive the condition for the maximum discharge through the chimney? And write the expression for maximum discharge. | 10M | CO-1 | BL-3 |
| UNIT-2 |  |  |  |  |  |
|  | Explain the working of a Low-level Counter flow jet condenser and evaporative condenser. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
|  | The surface condenser is designed to handle 16500 kg of steam per hour. The steam enters the condenser at 0.080 bar abs. and 0.86 dryness fraction and the condensate leaves the condenser at the corresponding saturation temperature. determine the rise in cooling water temperature, if the cooling water flow rate is 8.96 x 105 kg/hr. Assuming that the pressure is constant throughout the condenser. | 10M | CO-2 | BL-4 |
| UNIT-3 |  |  |  |  |  |
|  | Derive the condition for the maximum discharge through the nozzle? And write the expression for maximum discharge. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
|  | Steam enters a convergent divergent nozzle at 2 MPa and 4000 C with a negligible velocity and mass flow rate of 2.5 kg/s and it exits at a pressure of 300 kPa. The flow is isentropic between the nozzle entrance and throat and overall nozzle efficiency is 93%. Determine a) throat and b) exit areas. | 10M | CO-2 | BL-4 |
| UNIT-4 |  |  |  |  |  |
|  | The steam at 4.9 bar and 1600 C is supplied to a single stage impulse turbine at a mass flow rate of 30 kg/min, from where it is exhausted to a condenser at a pressure of 19.6 kPa. The blade speed is 300 m/s. The nozzle are inclined as 250 to the plane of wheel and the out blade angle is 350. Neglecting friction losses, determine a) power developed by the turbine, b) diagram efficiency, c) stage efficiency. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |  |
|  | What is compounding of steam turbine? Explain different types of compounding’s. | 10M | CO-3 | BL-3 |
| UNIT-5 |  |  |  |  |  |
|  | Explain the working principles of Vane type compressor and screw compressor. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
|  | A centrifugal compressor runs at 1440 rpm, compresses air from 101 kPa, 20o C and compresses it to a pressure of 6 bar isentropically. The inner and outer diameters of the impeller are 14 cm and 28 cm respectively. The width of the blades at inlet is 2.5 cm. The blade angles are 16o and 40o at entry and exit. Determine the mass flow rate of air, degree of reaction, power developed and width of the blade at outlet. | 10M | CO-3 | BL-4 |
| UNIT-6 |  |  |  |  |  |
|  | A gas turbine unit has a pressure ratio of 6 and maximum cycle temperature of 6100 C. The isentropic efficiency of turbine and compressor are 0.82 and 0.8 respectively. Calculate the power output in kW of an electric generator, geared to the turbine, when air enters the compressor at 150 C at a rate of 16kg/s. Take CP= 1.005 kJ/ kg. K and ϒ=1.4 for compression process and CP= 1.11 kJ/ kg. K and ϒ=1.333 for expansion process. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |  |
|  | Explain the working principle of solid and liquid propellant rocket engines. State their merits and demerits. | 10M | CO-4 | BL-3 |

***